INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR

USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

Title: Draft Decision Sheet - Ventilation of battery containers

This Draft Decision Sheet, prepared by BASEEFA, is issued for consideration

during the ExTAG Melbourne Meeting to be held 1st September 2009

Michel Brenon

ExTAG Secretary

| Address: SAI Global Building 286 Sussex Street Sydney NSW 2000 Australia | IECEx Secretariat Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 e-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: IEC 60079-11:2006 (Ed 5) IEC 60079-11:200X (Ed 6) | Clause: 7.4.2 3rd para |  |
| --- | --- | --- |
| Subject: Ventilation of battery containers Status of document: Draft | Key words: - Pressure - Container | # Date: 090814 Originator of proposal: BAS TC/SC involved: SC31G |
| Question - sub-clause 7.4.2 in IEC 60079-11: 2006 The 3rd paragraph requires the pressure inside the battery container to not exceed 30kPa (0.3 bar) above atmospheric. Does this paragraph apply for all types of battery i.e. primary and secondary? Answer Yes. The paragraph refers to battery containers, which could contain either primary or secondary cells, both of which could possibly vent hydrogen. Even if primary cells were considered not able to vent hydrogen, it is not known what happens to the m.i.e. of a gas mixture containing hydrogen and air (i.e. standard IIC gas) with another (possibly unknown) component from the gassing cells which is causing pressurisation. The purpose of this clause is to avoid lowering the ignition energy of the hydrogen/air mixture inside the container by preventing any increase in pressure above 30kPa above atmospheric, independent of the cell type. |  |  |