INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Title: Revision of ExTAG/136/CD – revision of ExTAG Decision Sheet

- DS 2004/006 - ExTAG Decision Sheets / Procedures for use of components on end products not covered by an ExTR

Circulation to: ExTAG Members

##### Introduction

During the ExTAG Paris Meeting, September 2008, it was agreed that more time was required for comment on ExTAG/136/CD Revision of ExTAG Decision Sheet DS 2004/006 ExTAG Decision Sheets / Procedures for use of components on end products not covered by an ExTR. Comment received has been circulated as ExTAG/159/CC.

This document, prepared by TestSafe in response to comment received, is an updated version of ExTAG/136/CD and issued for your consideration. Please submit comment via correspondence by

29th June 2009 to ExTAG Secretary, Mr. Michel Brenon, via

Christine Kane

If comments have not been received by the above date this document will be published as a revised ExTAG Decision Sheet.

Michel Brenon

Michel Brenon

ExTAG Secretary

| Address: SAI Global Building 286 Sussex Street Sydney NSW 2000 Australia | IECEx Secretariat Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 e-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

| Member Body | Clause/ Subclause | Paragraph Figure/ Table | Type of comment General/technical/ editorial | COMMENTS | Proposed change | Observation |
| --- | --- | --- | --- | --- | --- | --- |

Discussion points from TestSafe Australia:

Please find revised Decision sheet, attached as Annex A, relevant to the use of components, not covered by an IEC ExTR or an IECEx component CoC, on end-product, covered by an IEC ExTR.

Recently, TestSafe has reviewed a number of IEC ExTRs that included references

to ATEX certified components. However, the IEC ExTRs for the end-product did not contain the test data, material specifications, drawing references, etc, for these components.

TestSafe is of the opinion that only Ex components covered by an IEC ExTR, and subsequent IECEx Component CoC, can be referenced in an end-product IEC ExTR,

In all other cases, Ex components covered by a national or regional Ex conformity assessment Scheme (such as ATEX, ANZEx, CSA/FM/UL Approvals, etc), cannot be used in an end-product without the following steps being performed:

-  full Ex assessment of the component, including assessment of the drawings and original test data for compliance with the relevant requirements of the listed IEC standards .

- inclusion of the assessment and test data for the component in the IEC ExTR for the end-product. The relevant test report for the component must be referenced in the IEC ExTR for the end-product, and the test data must be in compliance with the requirements of OD009/Version 1, Annex A.

- inclusion of drawing references relevant to the component in the  IEC ExTR for the end-product.

-  inclusion of material specification data sheet references relevant to the component

in the IEC ExTR for end-product.

The above is required because the component contributes to the explosion protection of the end-product and, therefore, must be documented in the end-product IEC ExTR. The component drawings must form part of the end-product schedule drawings, or at least be linked or be traceable in some way to the end-product schedule drawings. In any case, the original tests and assessments performed on the Ex component must have been conducted by an IEC ExTL and performed in accordance with the requirements of OD009/Version 1, Annex A.

ANNEX A

## COLLECTION OF IECEx/ExTAG DECISIONS

| Subject: Procedures for use of components on end products not covered by an ExTR Status of document: Draft | Key words: - procedures - manufacturers - components | Date: 2009 05 25 Originator of Proposal: TestSafe |
| --- | --- | --- |

Question:

What procedures should be followed for manufacturers that are seeking an IEC ExTR (and subsequent IECEx CoC) on an end-product that includes Ex components not covered by an IEC ExTR and manufactured by another manufacturer?

| Answer: The guidelines below cover the following two situations: |  |
| --- | --- |
| Situation | Description |
| 1 | Ex components complying with a national or regional Ex conformity assessment Scheme, eg ATEX, with a test report issued by a body that is also an IECEx Test Laboratory (IEC ExTL) |
| 2 | Ex components complying with a national or regional Ex conformity assessment Scheme, eg ATEX, with a test report issued by a body that is NOT an IECEx Test Laboratory (IEC ExTL) |

# Guidelines for the issuing of an end-product ExTR

# For Situation 1

The IEC ExCB issuing the IEC ExTR for the end-product must request, and hold on file, a full copy of the test report/test data, drawings, datasheets, instructions, etc, for the Ex component, and verify, through review of this material (and possible re-test if deemed necessary), that all applicable IECEx construction and performance requirements have been met. The IEC ExCB must ensure that the following have been adequately addressed in the documentation submitted by the end-product manufacturer:

-The standards to which the Ex component has been tested are identical to the required IEC Standards for the end-product.

-The testing, as reported, is not ambiguous and specifically relates to the Ex component. 
-All tests were performed as required and in accordance with Annex A of OD 009 (ie any tests carried out by the component manufacturer were performed under the direct supervision of an IEC ExTL).
-Confirmation is received from the IEC ExTL that conducted the Ex component tests, that all tests were performed in accordance with IECEx procedures and ExTAG decision sheets.

Following review of the end-product manufacturer’s submitted documentation by the IEC ExCB, some additional testing may be required.

The IEC ExTR for the end-product must clearly show how the end product, when fitted with the Ex component, complies with the relevant standards. Therefore, all relevant clauses of the applicable standards must be adequately addressed. Ex component drawings must be included in, or linked to, the end-product schedule drawings.

# For Situation 2

The IEC ExCB issuing the IEC ExTR for the end-product must not accept the national or regional Ex certification, or the supporting test report, as the testing body who issued these documents has not been subjected to the formal IECEx assessment procedures that embody the well established practise of “Peer Assessment”. The IEC ExCB issuing the IEC ExTR for the end-product must conduct a full assessment of the Ex component drawings and carry out all necessary testing, as part of the IEC ExTR of the end-product.

# Guidelines for the issuing of a QAR

# For Situations 1 and 2

The IEC ExCB issuing the QAR for the end-product must ensure that the end-product manufacturer treats the Ex component manufacturer as a “critical item” supplier, especially if the component is critical to the Ex protection of the end-product. The following references are with this in mind.

Clause 7.4.1 of IECEx Operational Document, OD 005, refers to the need for the end-product manufacturer to control suppliers with specific requirements beyond those of ISO 9000:2000.

Clause 7.4.3 of OD 005, refers to the verification of components or assemblies supplied to the end-product manufacturer.

Annex A of OD 005 or (Annex B for sintered components), The guidelines of Annex A and/or Annex B also apply to the Ex component manufacturer. The end-product manufacturer must have a documented procedure for checking, on a regular basis (at least annually), that the routine tests and inspections, mentioned in Annex A, for the types of protection are conducted.  Verification by the end-product manufacturer may be by one of the following:

- The end-product manufacturer evaluating the procedures of the Ex component manufacturer for compliance with OD 005 and then conducting their own on-site assessment to ensure successful implementation of such procedures;

- Where the Ex component manufacturer holds ISO 9001 Certification by a body that is accredited by an IAF Member, the end-product manufacturer evaluates the procedures of the Ex component manufacturer for compliance with OD 005, with the ISO 9000 Certification agency providing a report that confirms implementation of such procedures.

The IEC ExCB issuing the QAR for the end-product has the responsibility of evaluating the effectiveness of the end-product manufacturer’s method of control over the Ex component manufacturer and, when there is a lack of evidence to demonstrate adequate control, the IEC ExCB has the right to carry out their own assessment of the Ex component manufacturer’s quality procedures.

-------------------------