INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SCHEME FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SCHEME)

- Title: Revision of ExTAG Decision Sheet DS 2004/006 – What procedures should be followed for manufacturers that are seeking an ExTR (and subsequent IECEx Certificate of Conformity, CoC) on an end-product that includes Ex components not covered by an ExTR and manufactured by another manufacturer?

Circulation to: ExTAG Members

##### Introduction

This document, prepared by TestSafe, is a draft revision of ExTAG DS 2004/006 –

What procedures should be followed for manufacturers that are seeking an ExTR (and subsequent IECEx Certificate of Conformity, CoC) on an end-product that includes Ex components not covered by an ExTR and manufactured by another manufacturer?

The document is issued on behalf of the ExTAG Secretary, Mr Michel Brenon, for consideration during the ExTAG Meeting Paris, September 2008, Meeting.

Michel Brenon

ExTAG Secretary

| Address: SAI Global Building 286 Sussex Street Sydney NSW 2000 Australia | IECEx Secretariat Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 e-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

Discussion points from TestSafe Australia:

Please find revised Decision sheet, attached as Annex A, relevant to use of component on end product not covered by an ExTR. 
During review of various IEC ExTRs, which included ATEX certified components, TestSafe have found that it is vital to cover the following:

- assess test data, drawings and material specification data sheets for the ATEX approved   Ex component. 
- include these assessments/tests in the end product IECExTR .
- include ATEX Ex component drawings in the end product certification drawings or linked ATEX Ex component drawings with the end product drawings.

The above is required because the Ex component contribute to the explosion protection of the end product and therefore it must be documented in the IECExTR and the Ex component drawings must form part of the end product certification drawings or at least be linked or be traceable in some way to the end product certification drawings.

ANNEX A

## COLLECTION OF IECEx/ExTAG DECISIONS

| Subject: Procedures for use of components on end products not covered by an ExTR Status of document: Approved | Key words: - procedures - manufacturers - components | ### Decision of meeting Date: 12 October 2004 Originator of Proposal: IECEx Secretariat |
| --- | --- | --- |

Question:

What procedures should be followed for manufacturers that are seeking an ExTR (and subsequent IECEx Certificate of Conformity, CoC) on an end-product that includes Ex components not covered by an ExTR and manufactured by another manufacturer?

| Answer: The guidelines below cover the following four situations: |  |
| --- | --- |
| Situation | Description |
| 1 | Ex components covered by an IECEx Test Report, ExTR |
| 2 | Ex components covered by an IECEx Certificate of Conformity, CoC; |
| 3 | Ex components not complying with 1 or 2 but are covered by another national or regional Ex conformity assessment Scheme, eg ATEX, by a body that is also an IECEx Test Laboratory, ExTL or IECEx Certification Body, ExCB |
| 4 | Ex components not complying with 1 or 2 but are covered by another national or regional Ex conformity assessment Scheme, eg ATEX, by a body that is NOT an IECEx Test Laboratory, ExTL or IECEx Certification Body, ExCB |

# Guidelines for the issuing of an ExTR

For Situation 1.

The ExCB issuing an ExTR for the end product should ensure that the ExTR, covering the Ex component, is valid and has not been withdrawn or cancelled by the issuing ExCB.  Checking with either the IECEx Website or contacting the ExCB that issued the ExTR may verify this.

Testing laboratory for IECEx TR for the end product  must perform the following:

- review IECExTR, and the documents referenced therein, for Ex component

- full assessment of  Ex equipment fitted with Ex component

- carry out  any additional test if required

- ExTR number for Ex component is listed in ExTR for Ex equipment

For Situation 2.

The ExCB issuing an ExTR for the end product should ensure that the CoC, covering the Ex component, is valid and has not been withdrawn or cancelled by the issuing ExCB.  Checking with either the IECEx Website or contacting the ExCB that issued the CoC may verify this.

Testing laboratory for IECEx TR for the end product must perform the following:

- review IECEx CoC  for Ex component

- review IECExTR, and document referenced therein,  for Ex component if the IECEx CoC does not include enough information to establish full compliance when installed in the Ex equipment.

- full assessment of  Ex equipment fitted with Ex component

- carry out  any additional test if required

- IECEx component certificate is  listed in ExTR for  Ex equipment

- Ex equipment certification drawings ( the end product drawings) must provide details for type/model/size/rating of Ex component, its interface and fitting to Ex equipment, its associated IECEx certificate number etc.

# For Situation 3

The ExCB issuing the ExTR for the end product, should request and hold on file, a full copy of the test report/test data, drawings and verify thru review of this material (and possible re-test if deemed necessary) that all applicable IECEx construction, performance requirements have been met, such as ensuring that:
-The standard to which the product has been tested is the same as the IEC Standard,

-The ExCB should ensure that the testing, as reported, does relate to the component submitted. 
-That all tests were done as required in accordance with Annex A of OD 005, i.e. any tests done by the component manufacturer were done under the direct control of the Test lab.
-Confirmation by the Test lab that conducted tests on the component that all tests were done in accordance with IECEx procedures and ExTAG decision sheets.

Following such review, by the ExCB, some top up testing may be required.

- ExTR for the end product must show how the end product when fitted with the Ex component complies with the relevant standards, including all relevant clauses being adequately addressed.

- Ex component drawings are included in or linked with Ex equipment certification drawings

# For Situation 4

Such results cannot be accepted, as the test body has not been subjected to the formal IECEx Assessment Procedures that embody the well established practise of “Peer Assessment”.

# Guidelines for the issuing of a QAR

# For Situation 2

The ExCB issuing a QAR should check to ensure that the end product manufacturer has a documented procedure for their checking, on a routine basis, that the IECEx CoC, covering the component, remains current.

# For Situations 1, 3 and 4

The ExCB issuing the QAR should ensure that the end product manufacturer treats the Ex component manufacturer just like any other supplier, especially if the component is a critical component to Ex. The following references are with this in mind.

Clause 7.4.1 of IECEx Operational Document, OD 005, refers to the need for the end manufacturer to control suppliers with specific requirements beyond those of ISO 9000:2000

Clause 7.4.3 of OD 005, refers to the verification of components or assemblies supplied to the end manufacturer.

Annex A of OD 005 or (Annex B for sintered components), The guidelines of Annex A and/or Annex B apply also to the Ex component manufacturer. The end product manufacturer should have a documented procedure for their checking, on a regular basis (at least annually), that the routine tests and inspections, mentioned in Annex A, for the types of protection are conducted.  Verification by the end product manufacturer may be by:

- The end product manufacturer evaluating the procedures of the component manufacturer, for compliance with OD 005 and then conducting their own site assessments to ensure successful implementation of such procedures;

- Where the component manufacturer holds ISO 9001 Certification by a body that is accredited by an IAF Member-

The end product manufacturer evaluates the procedures of the component manufacturer, for compliance with OD 005, with the ISO 9000 Certification agency, providing a report that confirms implementation of such procedures.

The ExCB issuing the QAR to the end product manufacturer has the responsibility of evaluating the effectiveness of the end product manufacturer’s method of their control over the component manufacturer and when there is a lack of evidence to demonstrate compliance, the ExCB has the right to carry out their own assessment of the component manufacturer’s quality procedures.

-------------------------