INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

IECEx OPERATIONAL DOCUMENT

Draft IECEx OD 033 Edition 1.0

Title: 	IECEx Operations Manual – IECEx Unit Verification Certificates

PROPOSED NEW OPERATIONAL DOCUMENT

IECEx Operational Document

IECEx ExMC agreed to prepare a draft Operational Document to provide for the issuing of IECEx Certificates under Unit Verification to cover specific items as produced and formed a small ExMC Ad Hoc WG consisting of Mr Brenon, Mr Olson and the ExMC Secretary.

This Draft Operational Document, prepared by the AdHoc WG for Unit Verification is now submitted for consideration by ExMC and sets out the procedures to be followed by ExCBs when processing applications from Ex Manufacturers, seeking IECEx Certification of individual apparatus or items. The resulting certificate, issued by the ExCB, will be an IECEx Unit Verification Certificate. This certificate is not transferrable should the equipment be sold or otherwise disposed-of.

IECEx Unit Verification Certificate applies to the specific equipment submitted for test only.  If similar equipment is to be certified, then a new Certificate will be required.

A listing of currently approved ExCBs is maintained on the IECEx Website www.iecex.com

ExMC Members are requested to submit any comments by 15 June 2009 to the IECEx Secretariat

Document History

| # Date | # Summary |
| --- | --- |
| 2009 XX | Original Issue (Version 1) |

| Address: IECEx Secretariat Standards Australia Building 286 Sussex Street Sydney NSW 2000 Australia |  | Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 Email: chris.agius@iecex.com |
| --- | --- | --- |

### Foreword

This document is supplementary to the Operational manuals and procedures operated by IECEx Certification Bodies (ExCBs), approved by the IECEx Management Committee to Issue IECEx Certificates of Conformity for individual equipment or apparatus operating in hazardous atmospheres where explosive gases may be present.

The IECEx Unit Verification Certificate enables certification of “one-off” or limited / specific production. An example is purpose built special equipment for use in hazardous atmospheres.

Each IECEx Unit Verification Certificate applies to specific piece of equipment only. Certification for a second piece of equipment of the same design, at a later stage requires a new application.

Production surveillance and quality assessment are not required for this form of certification.

The purpose of the Operational Document is to ensure that each ExCB, accepted by ExMC for the purposes of issuing IECEx Unit Verification Certificates, processes applications from Ex manufacturers with the same approach and technical rigour, known as “certifying the IECEx way”.

This IECEx Operational Document comprises 3 Sections:-

Section 1	Procedures for issuing an IECEx Unit Verification Certificate

Section 2	Repairs to equipment

Section 3	Procedures for the processing of applications for extension of scope to issue IECEx Unit Verification Certificates

The procedures are set out in table form identifying:

- Step number showing the link between flowcharts and table

- Description of the activity

- Related documents

- Responsible person or party

- Additional comments and remarks where appropriate

The preparation of this document has been done so with the aim of alignment with various ISO/IEC International Standards and Guides, including but not limited to the following:

Guide 28 	General rules for a model third-Party certification System for products

Guide 27 	Guidelines for corrective action to be taken by a certification body in the event of misuse of its mark of conformity

Guide 65 	General requirements for bodies operating product certification systems

SECTION 1

Procedures for the Issuing of an IECEx Unit Verification Certificate

This Section is to be applied by ExCBs when processing applications for an IECEx Unit Verification Certificate.

These steps are in line with the requirements of ISO/IEC Guide 65, General requirements for bodies operating product certification systems, in addition to the requirements as laid down in the IECEx Scheme rules, IECEx 02 for certified product.

Annex A contains a checklist of documentation required to fulfil Application and IECEx OD-017 requirements.

The following is a broad outline of how applications for IECEx Unit Verification Certificates are processed.

- The ExCB must review the Application.

- The documentation is reviewed (all the drawings, relevant datasheets, other data)

- A test report (ExTR) is prepared. As testing progresses, the ExTR results are compared with Technical Standards applicable to the equipment described on the Application. On completion of testing, the ExTR is issued.

- An IECEx Unit Verification Certificate is issued on-line

# Marking of products covered by IECEx Unit Verification Certificates

The marking for IECEx Unit Verification Certificates is as follows:

- Prefix to the certificate number is the usual ExCB issuing body prefix (letters)

- The next numbers are the year of the first issue of the certificate (eg 09)

- The certificate number consists of four (4) numbers

- Final part of the certificate number is “R”, which is added after the certificate number

Secretariat note:

Discussion is sought on the numbering system for IECEx Unit Verification Certificates, noting the need to be compatible with certificate numbering practices in place at present.

| Step | Section 1- Procedures for the Issuing of an IECEx Unit Verification Certificate of Conformity - Description of Activity | Related Documents | By Whom | Notes/Comments |
| --- | --- | --- | --- | --- |
| 1 | Ex Manufacturer receives a contract to design and build a “one-off”, usually for a special purpose or application. Standards for design should be specified in the original contract | IEC Standards National codes for electrical and mechanical safety | The applicant (or agent) |  |
| 2 | The design is approved by the Ex Manufacturer. The end user may review and approve the design before applying for IECEx certification. | All design documents | The Applicant (and their customer) |  |
| 3 | Product design documents are defined as a “package” | Design | The Applicant |  |
| 4 | Documentation covering the design is prepared. This includes datasheets, drawings and Ex approval information for any Ex-certified parts used in the product. | Design | The Applicant |  |
| 5 | Document review to requirements of IECEx document OD-017 | OD-017 | The Applicant |  |
| 6 | Documentation package has been reviewed by Applicant as being complete |  | The Applicant | The Manufacturer should contact the ExCB once documentation is complete |
| 7 | Application is submitted to the ExCB The application must be made to an ExCB that has been accepted by the IECEx Management Committee as an issuer of IECEx Unit Verification Certificates. Applicant Ex Manufacturers are encouraged to check the completeness of the application using the checklist in Annex B. Where the Ex Manufacturer is located in a non-IECEx participating country, the ExCB shall in conjunction with the ExMC Secretary, arrange for payment to the IECEx account of the surcharge, in accordance with OD 019 | IECEx 03 OD 019 (finance) | The Applicant | Applications for an IECEx Unit Verification Certificate of Conformity will verify compliance with the requirements of IECEx Scheme and relevant Operational Documents |
| 8 | Contract review is conducted by the ExCB receiving the application, in accordance with the ExCB's own Quality System and as required by ISO/IEC Guide 65. Contract review shall include: - A review to ensure that the application is within the scope of acceptance of the ExCB, eg protection techniques - Whether a surcharge applies for Service facilities from non IECEx member countries - Estimation of time and costs to complete project - Determination of any special requirements, eg travel for site audit etc. - Agreement on method and system of payment by applicant, in accordance with ExCB’s own policy and quality system | ExCBs own Certification procedures as included in their Quality System and ISO/IEC Guide 65 | ExCB | The results of the contract review shall be documented and recorded. Annex B can be used as a summary of the documents supplied. |
| 8a | ExCB shall only proceed where the contract review has been successfully completed |  | ExCB |  |
| 8b | Where unsuccessful, ExCB shall communicate in writing to the applicant with the applicant free to amend their application or select another ExCB. Change of ExCB’s may be required if the equipment to be certified is not covered by the ExCB scope of operation |  | ExCB |  |
| 9 | The product is prepared for testing. The ExCB will prepare a Test Plan as required. Use of a Test Plan ensures that all tests performed are logged, and they have the test acceptance criteria. Often, testing will involve a number of ExTR (test reports) due to the need to cover components, and the overall system. NOTE: The Test Plan is a controlled document. Plastic components are thermally conditioned before testing Test samples of critical items are submitted for destructive test Some components which are not certified may need test. | Test Plan | ExCB | A Test Plan may not always be required. Testing may follow the headings given in the ExTR (test report form) |
| 10 | The product is tested according to the Test Plan. Testing may be carried out by the ExCB or contracted to an ExTL (test laboratory). The Test Plan ensures that all tests are agreed and cover all certification issues. A record of all tests performed is kept in the Test Log. The tests performed are listed with pass/fail criteria and have provision for comments on the report. | Test Plan Test Log ExTR | ExCB or ExTL |  |
| 11 | Review of test results by ExCB Where product fails tests listed in the Test Plan, the Ex Manufacturer will be notified so that the product may be modified to pass the test which failed. For retested items, please refer to item 9 in this Guide. Failed tests which require correction or further action must be retested in full, to obtain a “pass” result in the ExTR. | Test Plan ExTR | ExCB monitors all testing. |  |
| 12 | Draft ExTR (Test Report) is issued. The interim Test Report must be approved by the Test Manager of the ExCB. The ExCB issuing the ExTR shall “Register” the ExTR on the IECEx “On-Line” Certificate of Conformity System. | ExTR | ExTL issues report ExCB registers the test report “on-line” |  |
| 13 | ExTR is approved by the ExCB test manager |  | ExCB |  |
| 14 | The draft IECEx Unit Verification Certificate is issued. All test documents, design data and design drawings are filed. Once satisfied that the test data and the supporting documentation are adequate the Certificate shall be issued, by the ExCB with the original sent to the applicant and a copy retained on the ExCB file. The registration process provides a database of information such as:: - Unique Certificate reference number or identifier - Name and address of Manufacturer of equipment covered by Certificate - Details of IECEx Unit Verification Certificate such as type of protection - Any comments that the ExCB feels should be noted | IECEx Unit Verification Certificate | ExCB certification panel reviews the draft certificate | ExCB shall be assigned a unique password to allow entry of the ExTR and certificate details. The data is entered “On-Line” Contact the IECEx Secretariat for any questions or concerns OD 011 Part 3 provides guidance to ExCBs on the registering of issued ExTR and also the compiling and issuing of On-Line Certificates of Conformity under the IECEx Unit Verification Program |
| 15 | The ExCB to whom the application for an IECEx Unit Verification Certificate was made shall conduct a certification review, in accordance with their quality management system, ensuring among others that: - All stages of the certification process have been documented and followed, including those contained in this manual - The ExCB have a signed commitment by the applicant to abide by the rules of the IECEx Scheme and ExCB’s certification conditions, eg agreement to have the ExCB undertake or arrange for surveillance audits - The applicant is aware of his/her obligations under the IECEx scheme. | IECEx 03 ExCB’s own quality management system | The person authorised to conduct such review within the ExCB to whom the original application was made | Contact the IECEx Secretariat for any questions or concerns |
| 15a | Where the review undertaken in step 15 is not successful, the ExCB management shall refer the matter to ExCB staff or applicant personnel as appropriate. |  | Management of the ExCB to whom the original application was made | Contact the IECEx Secretariat for any questions or concerns |
| 16 | IECEx Unit Verification Certificate shall be compiled using the IECEx “On-Line” system via the password protected system | Refer IECEx website OD 011 Part 3 | The person authorised to issue IECEx Certificates of Conformity within the ExCB to whom the original application was made | Contact the IECEx Secretariat for any questions or concerns |
| 17 | A draft of the IECEx Unit Verification Certificate is to be reviewed for errors. It may be beneficial to pass a draft copy to the applicant for them to assist in the final review prior to issuing the certificate. While the Draft Certificate is created “On-Line” the ExCB may create a protected PDF copy and send this to the Applicant, refer to OD 011 Part 3 for guidance. Every attempt shall be made to correct errors prior to issuing the certificate | Original application form submitted by the applicant. OD 011 Pt 3 | The person authorised to issue IECEx Certificates of Conformity within the ExCB to whom the original application was made | The Electronic version residing on the IECEx “On-Line” System remains the master version. Any printing or copying are considered “un-controlled” copies Contact the IECEx Secretariat for any questions or concerns |
| 18 | Certificate is issued via the IECEx “on-Line” registration system with the applicant being informed in writing by the ExCB, via letter, fax or E-mail |  | The person authorised to issue IECEx Unit Verification Certificate within the ExCB to whom the original application was made | Contact the IECEx Secretariat for any questions or concerns |

SECTION 2

Guidance concerning Repairs to certified equipment

This Section provides guidance to equipment owners and operators of equipment covered by IECEx Unit Verification Certificate.

The equipment owner is responsible for maintaining explosion protection of equipment.

Repairs should not compromise the original certification:

- Maintenance records should be kept by the Equipment owners. This record should be provided to show a history of repairs for the equipment.

- Repairs should be carried out by qualified personnel.
Repairs may affect certification.  All repairs should be assessed by a Competent Person to verify that the Unit Verification Certificate is still valid after the repair.

- Action to be taken by the equipment owner. The equipment owner should have records to show that the repaired item is safe to re-commission. In some countries, there may be additional requirements of legislation to address.

SECTION 3

Procedures concerning changes to products covered by 
IECEx Unit Verification Certificates

As Unit verification certificates covers specific items, any changes to products covered by an IECEx Unit Verification Certificate, that are not already provided for by the certificate, eg additional cable / conduit entries, shall require the issuing of new Unit Verification Certificate.

In such situations, it is recommended that the owner of the equipment, seeking product changes consults the equipment manufacturer, prior to undertaking any such modification.

ANNEX A

Documentation checklist for applications -
IECEx Unit Verification Certificate

The following is a list of specific documentation that an applicant needs to make available to the ExCB, when seeking IECEx Unit Verification Certification of individual equipment.

| - Applicant Name… Date… |  |
| --- | --- |
| - Details of applicant for Ex IECEx Unit Verification… Name, Country, details of design authority Legal status of company |  |
| - Formal application to ExCB (and contract for work), including… Description of the product being certified Address where the product was manufactured List of design standards Details of explosion protection technique List of critical Ex certified parts used Ex marking plate information and zone where product is going to be installed |  |
| - Test reports and final inspection being completed, including: a) test data for critical explosion protection parameters b) functional test results as appropriate |  |
| - Operation and service manuals describing how the product is going to be installed, and safely used. |  |
| - The applicant confirms that it is in full possession of all necessary rights on the product to be tested (including its components), insofar as testing and/or certification is applied for. |  |
| Signed: Dated: |  |