### INTERNATIONAL ELECTROTECHNICAL COMMISSION

IEC SCHEME FOR CERTIFICATION TO STANDARDS FOR

ELECTRICAL EQUIPMENT FOR EXPLOSIVE ATMOSPHERES

(IECEx SCHEME)

Title: Draft IECEx OPERATIONAL DOCUMENT- IECEx Rules of Procedure for testing at other locations (e.g. in manufacturers or users laboratories)

CIRCULATED TO: Ex Testing and Assessment Group (ExTAG) and ExMC

Introduction

This draft Operational Document is issued on behalf of the ExTAG Secretary, Mr Michel Brenon, at the request of the Convenor of ExTAG Working Group 6 - Rules of Procedure for testing at other locations  (e.g. in manufacturers or users laboratories, Dr Wolf Dill.

For background information, this Working Group was formed as a result of discussions held during the 2003 ExTAG Budapest Meeting. The Working Group first met in Budapest in 2003 and reported to the ExMC Budapest Meeting  (report contained in - ExMC/181A/RM - Clause 20. 1 Report from WG on Manufacturer’s testing). As result of this report additional experts were added to the Working Group which has subsequently worked by e-mail and prepared the following document for comment.

Comments from Members are invited and should be forwarded to the ExTAG Secretariat

by 09 August 2004 to:

Ms Christine Kane

christine.kane@iecex.com

| IECEx Secretariat C/- SAI Building 286 Sussex Street Sydney NSW 2000 Australia | Contact Details: Michel Brénon E-mail: michel.brenon@lcie.fr http://www.iecex.com |
| --- | --- |

Draft IECEx OPERATIONAL DOCUMENT

IECEx Rules of Procedure for testing at other locations 
(e.g. in manufacturers or users laboratories)

1	Preface – Mission Statement

- The procedures defined here should not be misunderstood. 
It is not the intention of the IECEx Scheme to replace third party testing by general acceptance of manufacturers tests. The high risks connected with the hazards of using electrical equipment in potentially explosive atmospheres need a very specific high level of competence and confidence.

- This paper must not encourage Ex-Testing laboratories (ExTLs) to reduce their laboratory equipment to cut their own costs and to use manufacturer’s laboratories instead. This would discredit the confidence in third party testing, while ExTLs would lose their competence due to lack of experience.

- The procedures outlined in this paper shall only be applied in exceptional cases in certain fields of testing Ex-equipment. Examples for typical fields of application are thermal testing of electrical motors, thermal endurance testing of plastic parts or tests that may be used in determining the ordinary location suitability of equipment or components used in hazardous locations. In these cases the following procedures may be used, but only if the ExTL in charge still continues to demonstrate technical competence by own testing, normally in the own laboratory.

### 2	Terms and Definitions

ExTL					Ex Testing Laboratory accepted by the IECEx Scheme

Other location	testing laboratory of other parties

(manufacturer, user, etc.) or

site suitable to be used for testing

Testing controlled			Testing is done by ExTL staff and staff of other parties,

by the ExTL				but ExTL staff is giving the orders and has responsibility.

Recurrent testing 			Testing at other locations is done on basis of a contract

at other locations			between ExTL and e.g. a manufacturer, because both

(Module 4)				parties will use this approach regularly or frequently.

### 3	General Rules

- It is a requirement that the ExTL conducting tests in other laboratories is accepted for the particular standard / type of protection and that the laboratory staff controlling or witnessing testing have the relevant competence and expertise for that standard / type of protection.

- The decision to conduct tests in other laboratories must be agreed to by the relevant ExCB.

- The ExTL shall remain in control and be responsible for testing and assessments conducted in other laboratories

- The ExTL wishing to conduct tests in other laboratories is responsible for:
- having ISO/IEC 17025 procedures adequately covering working off-site,
- assessing the facilities and equipment of the off-site laboratory and retaining records of the assessment,
- ensuring the presence of competent ExTL staff at the off-site laboratory,
- defining the role of ExTL staff in relation to the off-site laboratory,
- assessing the competence of other personnel involved in testing and
- concluding with the management of the off-site laboratory a contract or an agreement that   describes the activities and defines responsibilities.

- Tests conducted in other laboratories must be disclosed in the ExTR.

- The range and number of tests carried out in other laboratories shall not reduce the technical competence of the ExTL.

- Different modules for procedures as defined in the annexes to this document may be applied in cases where ExTL need to test in other laboratories.

- IECEx secretariat shall be informed about number and type of tests in other laboratories every year. The information shall be taken into account by IECEx assessors for assessments of ExTLs.

- Contracts about recurrent testing programs (Module No. 4) shall be notified to IECEx Secretariat.

4.	IECEx Rules of Procedure for testing at other locations: 
Table of Comparison of procedure modules

| Module No. | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Procedure | Ex-Testing controlled by ExTL at other locations | 100% witnessed Ex-Testing of specific aspects at other locations | Witnessed Ex-Testing at other locations, key phases of testing witnessed by ExTL | Recurrent testing of specific aspects of Ex-protection at other locations, based on a contract notified to IECEx |
| Abbreviation | CExT | W100ExT | WPExT | ROLExT |
| Test Plan to be made, approved and filed by | ExTL | ExTL (draft test plan may be developed by testing staff of other location) | EXTL (draft test plan may be developed testing staff of other location) | ExTL (draft test plan has to be developed in co-operation with testing staff of other location) |
| Presence of ExTL staff | 100% | 100% | during key phases of testing, at the discretion of ExTL, e.g. at the beginning and at the end of long term testing | during key phases of testing, at the discretion of ExTL, e.g. at the beginning and at the end of long term testing |
| Responsibility for calibration, accuracy and adjustment of test equipment and instruments not owned by ExTL, to be documented and filed by ExTL | ExTL | ExTL | ExTL, when present; other laboratory, when ExTL absent | management of other laboratory, under supervision of ExTL |
| Check of calibration label and accuracy | before each test by ExTL | before each test by ExTL | before each test as requested by ExTL | according to requirements given by ExTL |
| Directing of testing staff | staff member of ExTL | staff member of ExTL | staff member of ExTL | according to agreement between ExTL and management of other location |
| Responsibility for personal health and safety of laboratory staff | to be agreed between ExTL and management of other location | to be agreed between ExTL and management of other location | to be agreed between ExTL and management of other location | management of other location, according to agreement with ExTL |
| Test data records to be produced and signed by | ExTL | ExTL, may be based on notes and records taken by testing staff | ExTL, may be based on notes and records taken by testing staff | ExTL, may be based on notes and records taken by testing staff |
| Additional requirements for ExTR | Place of testing and use of non-ExTL owned test equipment to be disclosed in ExTR | Place of testing and identification of non-ExTL owned test equipment to be disclosed in ExTR | Place of testing and identification of non-ExTL owned test equipment to be disclosed in ExTR | Place of testing and IECEx-notified contractual basis to be disclosed in ExTR |
| Applicable requirements | ISO/IEC 17025 IECEx OD 009 | ISO/IEC 17025 IECEx OD 009 | ISO/IEC 17025 IECEx OD 009 | ISO/IEC 17025 & contract of ExTL with management of other location, notified to IECEx Secretariat |
| Proficiency testing |  |  | comparison tests between ExTL and other laboratory if deemed to be necessary | comparison tests between ExTL and other laboratory before start of test program; participation in proficiency testing programs of ExTLs (Note:(Proficiency testing programs for all ExTLs should, as needed, be established by a separate WG) |
| Typical examples | Big pressurized motors | Measurement of surface temperatures for T-Class | Long-term thermal ageing tests | Thermal testing of increased safety motors |
| Comments |  |  |  | 5-10% of the tests should be verified by comparative in-house testing of the ExTL. In the case of very large test objects (e.g. big motors) other precautions for verification might be necessary. |

*** End of Document ***