INTERNATIONAL ELECTROTECHNICAL COMMISSIONSCHEME FOR CERTIFICATION TO STANDARDS FOR EXPLOSIVE ATMOSPHERES

(IECEx Scheme)

Title: Draft ExTAG Decision Sheet - Non-metallic parts of enclosures

Circulated to: Ex Testing and Assessment Group (ExTAG)

INTRODUCTION

# This draft Decision Sheet is issued for consideration during the next ExTAG

# Meeting to be held in Brdo October 2004.

Michel Brenon

ExTAG Secretary

| Contact details: IECEx Secretariat 286 Sussex Street Sydney NSW 2000 Australia | Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 Email: chris.agius@iecex.com Web: www.iecex.com |
| --- | --- |

RECORD OF IECEx / ExTAG DECISION

| Standard(s) IEC 61241-0:2004 IEC 61214.1.1:1999 IEC 60079-0:2004 IEC 60079-0:2000 | Clause(s): 6.1.2 d), 6.1.4.1 5.1.4.1 7.1.1 7.1.2 d) 7.1.2 d) | Sheet No. |
| --- | --- | --- |
| Subject: Non-metallic parts of enclosures Status of Document: DRAFT | Keywords - elastomeric - O-rings | Decision of meeting Date: Originator of proposal:AU |

Question: Temperature Index (TI) for plastic materials

The above clauses require the Temperature Index (TI) for plastic materials. Does this apply to O-rings and elastomeric sealing materials of enclosures?

Answer:

These clauses do not apply to O-rings and elastomeric sealing materials of enclosures. These clauses apply to structural components of enclosures e.g. enclosure material (plastic) and polycarbonate windows. However, the thermal endurance tests shall apply to O-rings and elastomeric sealing materials of enclosures if the type of protection depends on it.