INTERNATIONAL ELECTROTECHNICAL COMMISSION SCHEME

FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SCHEME)

Title: ExTAG - Guidance on the preparation of IECEx Certificates and Reports covering more than one identifiable item of equipment

Circulated to: Ex Testing and Assessment Group (ExTAG)

##### INTRODUCTION

This paper, possible Decision Sheet, concerning the Guidance on the preparation of IECEx Certificates and Reports covering more than one identifiable item of equipment has been prepared by BASEEFA and is issued for discussion during the ExTAG Meeting, Shanghai, 5th September 2006

#### Michel Brenon

ExTAG Secretary

| Address: Standards Australia Building 286 Sussex Street Sydney NSW 2000 Australia | IECEx Secretariat Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 e-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

Guidance on the preparation of IECEx Certificates and Reports covering more than one identifiable item of equipment

Pre-amble

It has been noticed that there appear to be divergent practices between IECEx Certification Bodies in respect of the number of different products that can be covered on a single certificate.

This has a direct influence on the revenue of the scheme and implications for the easy management of certificates, including the frequency of raising the issue level of a certificate and the listing of ExTR documents on the certificate for each new technical issue.

Some certificates also refer to further documents (for example manufacturer provided reports or drawings) which are not directly controlled within the IECEx Scheme, or publicly available, for basic details of equipment within a range covered by a certificate.

The following is offered as a starting point for discussion at the next TAG Meeting, including a fundamental statement (open to debate) of the purpose of the description and other material in the certificate.  If the purpose of the certificate is not agreed, it is unlikely that further points can be decided in a meaningful manner.

Purpose of Certificate

The certificate is used by different people for different reasons.  The following is intended to capture all major uses:

1.	By potential purchasers, to search the IECEx Database for a suitable product.

2.	By the purchaser, to satisfy local regulatory bodies and insurers regarding an installation.

3.	By the manufacturer, to satisfy local regulatory authorities regarding compliance of a product.

4.	By the manufacturer, to satisfy his insurer regarding steps taken to avoid product liability claims (however expressed in different countries).

5.	By the manufacturer, as a marketing aid.

6.	By the manufacturer, to identify which products are covered by the certificate.

7.	By the certifier, to identify which products are covered by the certificate.

8.	By the IECEx Scheme, to promote the scheme.

9.	By the IECEx Scheme, as a source of revenue.

10.	By the manufacturer, purchaser, installer and user of the equipment to transfer specific information needed for installation and use.

For most of these purposes, except perhaps items 4 and 5, the ideal situation is a one to one correspondence between each discrete design of equipment and its certificate.  However, it is acknowledged that certification is an expensive business and that costs can be saved by covering more than one discreet design (in other terms, a range of equipment) on a single certificate.

There therefore needs to be a compromise whereby logical ranges of equipment can be adequately described and covered in a single certificate.

Unless the extent of the range is limited by logic, it would be possible to have a single certificate covering the entire output of a particular factory including, perhaps, junction boxes, luminaires and solenoid valves !  Such a certificate would be unlikely to satisfy any of the purposes of a certificate except, maybe, item 4 above.

Content of Certificate

OD 011 Part 2 Version 3.1 contains suggestions for the content of the boxes headed “Electrical Apparatus” and “Equipment” at paragraphs 2.7 and 2.20 respectively.

Paragraph 2.7 suggests that if there is insufficient space then the words “Refer to Schedule” can be inserted.

This may be valid on some extreme occasions but certainly does not assist fulfilment of any of the purposes listed above.

It is necessary that the effective title of the certificate (which will be the principle search item in the database) should be as clear and succinct as possible.  This does not preclude using the word “Range” in the title, but the rest of the title should put at least an approximate limit on the range.

Paragraph 2.20 refers to “…. the detailed information is entered to clearly describe what is covered by the IECEx CoC”.

At this point sufficient information must be given such that it is clear whether or not a given physical object is likely to be the subject of the certificate.  The complete constructional details are only given in the manufacturer’s drawings, listed within the ExTR, but the written description in the certificate should not leave room for doubt with regard to the coverage of the certificate.  This would include, for example, details of all principal ratings of products within a certified range.

A generalised description, for example, “An enclosure containing various items of control equipment”, would not allow the reader of the certificate to judge what is actually within the scope of the certificate.

Conversely, “An Ex d control station comprising a rectangular enclosure with plain flanged bolted lid, size A x B x C mm, containing up to 7 relays, 4 transformers, 2 contactors, …… , together with associated rail mounted terminals.  The lid is provided with up to 4 pushbutton actuators, ….. , and up to 2 windows for internally mounted displays.  Maximum rating 250 V, 10A.  Drilled and tapped holes are provided for the accommodation of up to 6 M20 suitable Ex d cable glands separately certified as equipment ( not as a component)”, is totally clear and meets all required purposes.

In this case, stating the maximum number of each internal item allows the manufacturer to substitute similar devices and/or omit some of the internal items,  so that the product may fulfil a similar, but non-identical, function, whilst still clearly providing the necessary traceability between the certificate and the product.

“Range” certification can be properly applied to similar products where a principal part of the descriptive material remains constant.  Thus a family of twin and single tube fluorescent luminaires in the same basic form of enclosure is logical, but to extend the certificate to cover both “overtube” and “diffuser” styles of enclosure would not be appropriate.  A singe design of housing containing appropriate control gear for three different types of high power discharge lamps, together with different reflectors to match the optical needs of the different lamp forms would also form a logical range.

The “principal part of the descriptive material remaining constant” does not necessarily mean that common components need be used throughout.  For example a logical range of Ex e enclosures to be certified empty as Components might all be different sizes, but they would have common features for lid fastening, gasket retention and for mounting through the base.  Alternative materials (for example high temperature gaskets) would be available throughout the range

Problems with “extended” Ranges

All certificates for products having an extended production life go through a process of change as the product line matures.  This may be by reason of changed availability of manufacturing parts (particularly electronic components) or by improvements in design or adaptations to suit customer requirements.  Each change requires a new edition of the certificate, together with the associated ExTR (either full or abbreviated, depending on the extent of the modifications).

The broader the range of equipment on a single certificate, the greater the number of changes that will happen.  This gets particularly complex when a change only applies to a very restricted part of the range.  After a series of such changes, the certificate will have grown extensively in the number of pages, only some of which will now apply to any one individual product.  Such a certificate becomes very difficult for the manufacturer, certifier and customer alike to manage in a sensible way.

Ideally, the logical range should be such that changes to the certificate are likely to apply to all products.

The whole purpose of component certification is to facilitate the certification of actual equipment such that it can be meaningful and yet relatively quick and cost effective to produce. For example by obtaining a component certificate for an Exe or Exd enclosure the manufacturer can readily obtain a new certificate for each principle build that satisfies all the objectives described earlier.  Neither vague initial certification, that effectively leads to self certification by the manufacturer, nor endless additions to a single certificate satisfy these requirements.

References for Ratings

It should not be necessary to refer to “private” documents to establish basic ratings of products.  Simple ratings may be given in the text of the description, or even in the headline of the equipment title.

Where more complex ratings are assigned (for example the terminal parameters for intrinsically safe equipment), it is appropriate for them to be given either on the sheet “additional information”, or in an Annex.

Because of the difficulty of formulating tabulated or table text in the HTML text blocks, it is possibly advantageous to use an Annex.

Length of Descriptive Text

In order to ensure that satisfactory prints can be made of on-line certificates, it is desirable that the basic description should not cause the text block to expand as this will result in the physical printed pages not matching the pagination printed on them.

If an extended description is necessary, an abbreviated description should be provided at item 2.20, with a reference to a full description on the “additional information” page or in the Annexe.

Although there will be a few exceptions, If the description of a “range” of products has to extend beyond a single additional html page, or two pages in an annex, it is probable that the range is too extensive for a single certificate.