INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR

CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulated to: ExTAG – IECEx Testing and Assessment Group

TITLE: Compilation of comments and observations on ExTAG/318/CD -: Draft ExTAG Decision Sheet – Marking of Ex t equipment with and without layers of dust

INTRODUCTION

This document is a compilation of comments and observations from the originator Mr Nicholas Ludlam, FM Approvals Ltd on ExTAG/318/CD-: Draft ExTAG Decision Sheet – Marking of Ex t equipment with and without layers of dust.

A revised document has been prepared taking into account these comments and observations, ExTAG/318A/CD ExTAG Draft Decision Sheet – Marking of Ex t equipment with and without layers of dust, which has been issued for discussion during the ExTAG Meeting being held in The Hague.

Julien Gauthier

ExTAG Secretary

| Address: Level 33, Australia Square 264 George Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 46 28 4690 Fax: +61 2 46 27 5285 e-mail: chris.agius@iecex.com http://www.iecex.com |
| --- | --- |

| Member Body | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation |
| --- | --- | --- | --- | --- | --- | --- |
| EXA HR |  |  | G | Current standards define marking of equipment for Group III for both cases, with or without mark of dust layer, and we consider that is satisfactory for IECEx certification. Marking can be additionally explained in CoC and manufacturer’s instruction for use if necessary. No need for ExTAG Decision Sheet on this subject and we disagree with proposal for ExTAG DS. | Withdraw proposed ExTAG DS | This draft DS was prepared to align with other IEC 60079-x standards and to address the specific condition when a dust layer is present. IEC 60079-0 Ed 6.0 section 29.5 d) implies that for EPL Da the surface temperature marking includes an indication of the thickness of the layer of dust, but does not specifically state this. |
| FMG US |  |  | General | FM Approvals LLC (FMG) supports the decision as drafted. | None | Noted |
| NANIO CCVE RU |  |  | General | We support this ExTAG/318/CD except that the term ‘temperature class’ is not applied to Group III equipment | The term ‘temperature class’ shall be changed to ‘maximum surface temperature’ in the text of this draft. | Accept |
|  | Draft answer 5th paragraph |  | Editorial |  | For Level of Protection ‘ta’ a dust layer is always used in the marking | Accept in principle. This relates to the how the maximum temperature is obtained. Change to; “For Level of Protection ‘ta’ a dust layer is always used to determine the maximum surface temperature and therefore for equipment with dust protection by enclosure ‘t’;….” |
| NEPSI CN |  |  | editorial | NEPSI approves the draft in general, but the last sentence in last paragraph seems to be repeated. | - Delete sentence “In the case for EPL Db or Dc, tested with a dust layer, The maximum surface temperature without the dust layer is not required to be marked. ” | Accept in principle. Changed sentence to a separate bullet point and highlighted the word ‘without’ in the first bullet point. |
| TUR DE | all |  | General | We support the proposed answer. |  | Noted |
| UL US |  |  |  | UL –USA supports this DS. |  | Noted |
| VTT FI |  |  | T | The original purpose of that 200 mm (old 500 mm) dust layer test was allow the use of equipment in places with uncontrolled dust thicknesses and this test is a must for EPL Da. If we now mark ta- equipment with that 200 mm layer thickness, it will be misunderstood that max allowed layer thickness is that 200 mm. | Apply that TL-marking only for tb- and tc-equipment and when tested with 200 mm or greater dust thickness then it will be needed explanation behind X-condition that equipment is allowed to use in places with uncontrolled dust thicknesses when the ignition temperature of dust layers is equal or greater that TL+ 75 K. | Not accepted. IEC 60079-31 requires that the TL marking is required in all cases when the apparatus is tested with a dust layer. Agree that additional application information should be considered for IEC 60079-14 Ed 6.0 |