INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulation to: ExTAG – IECEx Testing and Assessment Group

Title: Draft OD 010-2 – Guidance for the Compilation, Issuance and Receipt of IECEx Test Report (ExTR) Packages

INTRODUCTION

This document, a revised edition of ExTAG/192A/CD, has been by ExTAG WG1, prepared taking into account comments received, including those during the ExTAG Berlin meeting.

In accordance with ExTAG procedures this proposed document is being circulated for consideration by members. Should you have any comments please submit them by -

2011 08 25 to the ExTAG Secretary, Mr. Michel Brenon, via

Christine Kane

## Michel Brenon

ExTAG Secretary

| Address: IECEx Secretariat 286 Sussex Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 E-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

Draft OD 010-2 – Guidance for the Compilation, Issuance and Receipt

of IECEx Test Report (ExTR) Packages

# Introduction

The purpose of this Operational Document is to establish a common approach amongst all IECEx Test Laboratories (ExTLs) and IECEx Certification Bodies (ExCBs) operating within the IECEx System regarding the following:

- Compilation, Approval and Issuance of ExTR packages by Issuing ExTLs/ExCBs; and

- Receipt, Review and Audit of ExTR packages by Receiving ExTLs/ExCBs.

The term “ExTR package” refers to the overall record of the evaluation and testing performed on a given item or product. An ExTR package is comprised of an ExTR Cover and one or more associated ExTR documents.

The term “ExTR document” refers to any of the IECEx form documents that include the individual Ex Test Reports for each of the IEC Ex standards, along with the ExTR Addendum and ExTR of National Differences documents (in addition to the ExTR Cover).

The term ‘blank’ in reference to an ExTR document refers to the form version of that ExTR document that is posted on the IECEx website for ExTL and ExCB use when evaluating and testing IEC Ex equipment and components.

Regarding the development and posting of new or revised blank ExTR documents by ExTLs and ExCBs, see OD 010-1, Guidance for the Development and Posting of Blank IECEx Test Report (ExTR) Documents.

This Operational Document (OD) 010-2, together with OD 010-1, complement the IECEx Operations Manual, OD 009. Terminology has been aligned to comply with IECEx 02, 4th edition.

OD 010-2 is issued as a result of discussions and comments prior to, during and subsequent to the 2009 and 2010 IECEx ExTAG meetings. Substantial changes made since the 1st edition of OD 010 impact all aspects of the document.

# Compilation, Approval and Issuance of ExTR packages

## General Rules

An ExTR package is comprised of an ExTR Cover and one or more associated ExTR documents. These associated ExTR documents may include individual Ex Test Reports for one or more of the IEC Ex standards, ExTR Addendums and/or ExTR of National Differences.

The test results presented in an ExTR package relate only to the item(s) or product(s) evaluated and tested. If there is a need for the entry of data or additional details beyond the standard fields of an ExTR package, the following applies:

- "(see Attachment #)" refers to additional information appended to any ExTR document.

- "(see appended table)" refers to a table appended to this document.

The ExTR Reference number that is generated by the IECEx “On-Line” Certificate System for a given ExTR package is to be added to the top of each page of each associated ExTR document.

Throughout an ExTR package, a point “.” is used as the decimal separator.

Where the term “N/A” appears in any part of an ExTR package, it indicates that the associated issue was considered “Not applicable” to the involved evaluation.

While the content intended to be entered in some ExTR document fields is self-explanatory, other fields could benefit from additional explanatory commentary.  Embedded comments are included as part of many ExTR document fields, and these fields will be further explained as part of this OD.

Any embedded comments provided to assist with the compilation of ExTR documents must be removed prior to final approval by the involved ExCB.

## Copyright and reproduction

The content for all blank ExTR documents (i.e. the form text and layout) are under copyright ownership © 2011 by the International Electrotechnical Commission System for Certification to Standards Relating to Equipment for use in Explosive Atmospheres (IECEx System), Geneva, Switzerland. All rights reserved.

This form text and layout of blank ExTR documents may be reproduced in whole or in part for non-commercial purposes as long as the IECEx System is acknowledged as copyright owner and source of the material. The IECEx system takes no responsibility for, and will not assume liability for, damages resulting from the reader's interpretation of the reproduced material due to its placement and context.

The technical content of completed ExTR documents shall not be reproduced except in full without the written approval of the Issuing ExCB and ExTL.

## ExTR Cover

### General

An ExTR Cover is the sole top-level document to associate together all other parts of an ExTR package.

The purpose of the ExTR Cover for an ExTR package is to contain key information that is common across all other parts of the ExTR package. This eliminates the need to repeat such common information across multiple ExTR documents, and highlights this key information in a standardized fashion in one location for ready access.

All ExTR package documents are compiled and reviewed by the ExTL. The Issuing ExCB indicates final approval of the overall ExTR package on the ExTR Cover.

In addition to the explanatory commentary that is provided in this OD regarding certain fields in blank ExTR Covers, an example of a completed ExTR Cover is included in Annex A.

### ‘Standards associated with this ExTR package’ field

The ‘Standards associated with this ExTR package’ field is for identifying the standard number and edition or year of issue for all IEC Ex standards associated with the ExTR package.

### ‘Clauses considered’ field

The ‘Clauses considered’ field is for indicating if all clauses from the associated IEC Ex standards were considered and commented on in the associated Ex Test Report(s).

- If all clauses were considered (even if some were deemed ‘N/A’), the text ‘All clauses considered’ should be entered in the field.

- If all clauses were not considered, the text ‘Only specific clauses considered’ should be entered in the field.

Note: The second bulleted option above is primarily only applicable when an Ex Test Report of Partial Testing is being assembled by one ExCB to address no more than a couple specific tests or constructional issues that will then be submitted for use by another ExCB.

### ‘Model/type reference’ field

The ‘Model/type’ reference’ field is for indicating the overall model number or series for the item(s) or product(s) covered by the ExTR package.

- ‘*’ is used to indicate variable fields.

- ‘General Product Information’ field is to be used for specific nomenclature details.

### ‘All testing fully performed by ExTL staff at ExTL address above’ field

The ‘All testing fully performed by ExTL staff at ExTL address above’ field is for indicating whether all testing was fully performed by ExTL staff at the ExTL address noted earlier in the ExTR Cover, or if some testing was fully or partially witnessed by the ExTL staff in accordance with OD 024, IECEx Rules of Procedure covering testing, or witnessing testing at a manufacturer’s or user’s facility.

- Select 'Yes' if all testing was fully performed by ExTL staff at the ExTL address noted earlier in the ExTR Cover.

- Select 'No, See below for additional details' if some testing was fully or partially witnessed by the ExTL staff in accordance with OD 024.

- Delete the response not selected, along with the parenthesis.

There is a field later on in the ExTR Cover for providing additional details if some testing was fully or partially witnessed by the ExTL staff in accordance with OD 024.

### ‘Rated service temperature range (°C) for Ex Components’ field

The ‘Rated service temperature range (°C) for Ex Components’ field is for indicating the rated service temperature range when the ExTR package addresses an Ex Component.

This row is to be removed if the ExTR package does not address an Ex Component.

### ‘General remarks’ field

The ‘General remarks’ field contains some fixed text that is not intended to be altered.

### ‘Copy of Marking Plate’ field

The ‘Copy of Marking Plate’ field is for inserting a copy of the Marking Plate for the item(s) or product(s) covered by the ExTR package. A copy need not be included if the ExTR package is not supporting an IECEx CoC.

For instances in which a copy of the Marking Plate is not included, a statement is to be entered indicating the reason for the exclusion (such as ‘Marking plate content not reviewed, as CoC not issued in conjunction with this ExTR’).

### ‘General product information’ field

The ‘General product information’ field is for including any nomenclature details, such as a description of the type designations, if applicable.  This is a free-form field for detailing any information that is considered pertinent by the involved ExTL/ExCB regarding the involved item(s) and product(s) and the associated evaluation and testing.

### ‘In accordance with OD 024, testing not fully performed by ExTL staff at the above ExTL address’ field

The ‘In accordance with OD 024, testing not fully performed by ExTL staff at the above ExTL address’ field is for itemizing each test that is not performed by ExTL staff at the ExTL address noted earlier in the ExTR Cover…

- Indication to be provided of who performed and/or witnessed the test, and where the testing was performed and/or witnessed.

- If any electronic medium such as video was used for “live” witnessing of any testing, this shall also be detailed.

- Confirmation to be provided that all such testing was in accordance with all OD 024 procedures.

If all testing was performed by ExTL staff at the ExTL address noted earlier in the ExTR Cover, “N/A” to be indicated for not applicable.

### ‘National differences considered as part of this evaluation, if any’ field

The ‘National differences considered as part of this evaluation, if any’ field is for indicating if National differences were considered, including general details regarding how they were addressed.

If National differences were consideredhe associated standards were considered.000000000000000000000, explain in general why a Test Report of National Differences is or is not included. The following is an example of text that could be entered to explain why the EN National Differences to IEC 60079-0:2004, IEC 60079-11:2006 and IEC 60079-26:2006 were considered as part of the evaluation and testing of an item or product, but no ExTR of National Differences were issued:

“The change to the product involving the optional housing was reviewed with respect to the National Differences for EN 60079-0:2006, EN 60079-11:2007 and EN 60079-26:2007 and found to be in compliance with these requirements.

No ExTRs of National Differences were issued because:

- There are no National Differences between IEC 60079-11:2006 and EN60079-11: 2007.

- The optional housing does not affect any National Differences between IEC 60079-0:2004 and EN 60079-0:2006.

- The optional housing does not affect any National Differences between IEC 60079-26:2006 and EN 60079-26:2007.”

If no National Differences were considered, ‘N/A’ is all that needs to be entered in this second field.

These fields are not for the actual recording of any specific “Result – Remark” details regarding any specific National Differences. The ExTR of National Differences is the document for recording any specific “Result – Remark” details.

### ‘Conditions of Use for Ex Equipment or Schedule of Limitations for Ex Components, if any’ field

The ‘Conditions of Use for Ex Equipment or Schedule of Limitations for Ex Components, if any’ field is for detailing any required “Specific Conditions of Use” for Ex equipment or “Schedule of Limitations” for Ex Components from any of the associated Ex Test Report, Addendum or National Differences documents that comprise the ExTR package.

If there are no required “Specific Conditions of Use” or “Schedule of Limitations”, then “N/A” is to be entered in this field.

### ‘Routine tests, if any’ field

The ‘Routine tests, if any’ field is for detailing any required Routine tests from any of the associated Ex Test Report, Addendum or National Differences documents that comprise the ExTR package.

If there are no required Routine tests, then “N/A” is to be entered in this field.

## Ex Test Report

### General

Each Ex Test Report is part of an ExTR package that may include other Ex Test Report, Addendum and National Differences documents, along with a single ExTR Cover.

The purpose of an Ex Test Report is to provide a standardized clause-by-clause documentation of the evaluation and testing that verified initial compliance of an item or product with an IEC Ex standard. This standardized format consists of one or more checklist tables and additional narrative content as applicable.

Note: Supplemental evaluation and testing to address alternate constructions or modifications are documented as part of an ExTR Addendum. See ExTR Addendum details below.

All ExTR package documents are compiled and reviewed by the ExTL. The Issuing ExCB indicates final approval of the overall ExTR package on the ExTR Cover.

In addition to the explanatory commentary that is provided in this OD regarding certain fields in blank Ex Test Reports, an example of a completed Ex Test Report is included in Annex B.

- Checklist table(s)

### General

The checklist table portion of an Ex Test Report is referred to as the “test case” section. Each “test case” section is formatted into four columns of a single table beneath an overall heading indicating the involved IEC Ex standard.

The first two columns in each Ex Test Report checklist titled “Clause” and “Requirement - Test” are provided for reference, and should not be modified by the ExTL. For the complete text of the requirement, the ExTL should reference the involved IEC Ex standard.

The next column in each Ex Test Report checklist titled “Result - Remark” is to be completed as part of the evaluation and testing. In this column, a measured value or an explanation shall be entered to detail the results from the “Requirement – Test” column.

The “Result - Remark” column in each Ex Test Report confirms the compliance or declares the particular clause as not applicable (the verdicts can be either “Pass” or “N/A”). There are different “test case” types, and the ExTL should pay attention to the type of the clause in order to correctly complete the ExTR package.

### ‘Test case’ types

Different “test case” types may be presented in one of the following ways (each type is followed by an example):

- Numbered Heading – No associated text: No “Result – Remark” or “Verdict” is applicable since no requirements are associated with the numbered heading in the IEC Ex standard.

| IEC _____-__ |  |  |  |
| --- | --- | --- | --- |
| Clause | Requirement – Test | Result – Remark | Verdict |
| 5 | Flameproof joints |  |  |

- Numbered Heading – Normative requirement: Both “Result – Remark” and “Verdict” are applicable since requirements are associated with the numbered heading in the IEC Ex standard.

| IEC _____-__ |  |  |  |
| --- | --- | --- | --- |
| Clause | Requirement – Test | Result – Remark | Verdict |
| 9 | Light-transmitting parts |  |  |

- Numbered Heading – No associated Normative text: No “Result – Remark” or “Verdict” are applicable since text is only Informative and not Normative.

| IEC _____-__ |  |  |  |
| --- | --- | --- | --- |
| Clause | Requirement – Test | Result – Remark | Verdict |
| Annex F (Informative) | Mechanical properties for screws and nuts |  |  |

Numbered headings involving Normative requirements, such as b) above, are referred to as regular “test case” clauses. Regular “test case” clauses shall include explanatory details under the “Result – Remark” heading which support the Verdict being entered.

Some regular “test case” clauses make it mandatory that specific details be entered under the “Result – Remark” heading in support of the “Verdict”. A dotted leader as shown in the example below identifies such instances.

| IEC _____-__ |  |  |  |
| --- | --- | --- | --- |
| Clause | Requirement – Test | Result – Remark | Verdict |
| 5.2.1 | Width of joints (L).................: |  |  |

### ‘Result – Remark’ column

The “Result – Remark” column is for explanatory details that result from the related “Requirement – Test”, and that support the ultimate “Verdict”. When entering “Result – Remark” field comments, such comments shall clearly indicate why the particular item passed or is not applicable. Examples of such details include a measured value, a test result or an explanatory remark.

To facilitate acceptance by Receiving ExCBs without re-testing, Ex Test Reports shall provide clear, concise and sufficient information supporting the Issuing ExCBs evaluation and testing. Ex Test Reports should include any additional information that will help the Receiving Body. Examples of such additional information include photos, complete data sheet packages, schematics, installation documents and detailed component information.

If a clause in the Ex Test Report checklist involves a number of sub-clauses, and nothing under the clause or its related sub-clauses are applicable to the involved evaluation and testing, the following actions may be taken so as to simplify the completed Ex Test Report:

- In the “Result – Remark” field for the main clause, a comment is to be entered indicating the reason for the entire clause not being applicable;

- In the “Verdict” field for the main clause, “N/A” is to be entered; and

- All the rows for the related sub-clauses to the main clause are to be deleted.

- ‘Verdict’ column

The “Verdict” column is for stating the judgment resulting from analysis based on the “Requirements – Test” column, and reflective of the explanatory details under the “Result – Remark” column.  Allowed verdict choices are: Pass, or N/A (Not applicable).

### Measurement section

In addition to the checklist portion of an Ex Test Report, there is often a need to detail measured values and test results that do not easily fit in the fields under the “Result – Remark” column.

To address such situations, this area is provided for additional narrative remarks and tables of measured values, specific test conditions and test results.

## ExTR of National Differences

Each ExTR of National Differences is part of an ExTR package that may include other Ex Test Report, Addendum and National Differences documents, along with a single ExTR Cover.

An ExTR of National Differences is to supplement an Ex Test Report or ExTR Addendum, with a separate ExTR of National Differences document issued for each intended country or region. All National Differences of the intended country or region are to be tabulated and remarked upon as part of this document, even those that are not applicable.

Similar to the function of an Ex Test Report, an ExTR of National Differences provides a clause-by-clause documentation of the evaluation and testing that verified compliance of an item or product with an Ex standard. However, while an Ex Test Report (and ExTR Addendum) document compliance with an IEC Ex standard, an ExTR of National Differences documents compliance with a local country or regional Ex standard.

Note: See Ex Test Report details above for related guidance when compiling, approving and issuing an ExTR of National Differences.

The standard number for the local country or regional Ex standard for which National differences are being addressed should be added in a header row at the top of the checklist portion of the ExTR of National Differences.

An ExTR of National Differences is to be compiled and reviewed by the ExTL. The Issuing ExCB indicates final approval of the ExTR of National Differences as part of the overall ExTR package on the associated ExTR Cover.

In addition to the explanatory commentary that is provided in this OD regarding certain fields in blank ExTR of National Differences, an example of a completed ExTR of National Differences is included in Annex C.

## ExTR Addendum

## General

Each ExTR Addendum is part of an ExTR package that may include other Ex Test Report, ExTR Addendum and ExTR of National Differences documents, along with a single ExTR Cover.

The purpose of an ExTR Addendum is to document compliance of an alternate construction to a product already covered by an existing ExTR package. An ExTR Addendum is to supplement a previously issued ExTR package (which may include other previously issued ExTR Addendums).

Only those clauses applicable to the supplemental issue being addressed are to be tabulated and remarked upon as part of this document. An ExTR of National Differences may also supplement this document.

Supplemental modifications that could require an ExTR Addendum include:

      - Design changes impacting the method of protection, such as…

- Changes to the electrical rating

- New or alternate components added

- Enclosure material / thickness changes

- Circuit layout changes

      - Addition of model/type numbers as part of a product series

      - Name/address changes regarding Applicant/Manufacturer

      - Corrections to the original test report documentation

Similar to the function of an Ex Test Report, an ExTR Addendum provides a clause-by-clause documentation of the evaluation and testing that verified compliance of an item or product with an IEC Ex standard. The difference between the two is that the Ex Test Report provides documentation of the initial evaluation and testing of an item or product, while an ExTR Addendum provides documentation of a supplemental modification to an item or product.

Note: See Ex Test Report details above for related guidance when compiling, approving and issuing an ExTR Addendum.

The test results presented in an ExTR Addendum are only valid when considered together with the related Ex Test Report that was previously issued (along with any other previously issued ExTR Addendums for the same item or product).

When an item or product is subject to supplemental modification, the “General product information” field under the associated ExTR Cover for the ExTR Addendum shall clearly identify the nature of such modifications and corrections, as applicable. Regarding the other fields under the associated ExTR Cover, the content to be entered need only comment on the supplemental modification issue. There is not a need to repeat all the content from the initial evaluation and testing.

An ExTR Addendum is to be compiled and reviewed by the ExTL. The Issuing ExCB indicates final approval of the ExTR Addendum as part of the overall ExTR package on the associated ExTR Cover.

In addition to the explanatory commentary that is provided in this OD regarding certain fields in blank ExTR Addendums, an example of a completed ExTR Addendum is included in Annex D.

## ExTR Addendum vs New Ex Test Report

There are two approaches to addressing a supplemental modification that is submitted to a previously issued Ex Test Report. One approach is to issue an ExTR Addendum. The other approach is to issue a new Ex Test Report that incorporates all content of any previously issued Ex Test Reports, together with any previously issued ExTR Addendums and the latest supplemental modification.

It is at the discretion of the Issuing ExCB as to which approach to follow for any supplemental modification submittal. Unless the modifications are of a comparatively minor nature, it is recommended that no more than three modifications be allowed to a previously issued ExTR package. After the third modification, a new ExTR package should be issued that incorporates all content of any previously issued Ex Test Reports, together with any previously issued ExTR Addendums and the latest supplemental modification.

The rationale behind limiting the number of modifications to a previously issued ExTR package is to avoid confusion regarding what is actually covered by the combination of the Ex Test Report and any subsequent ExTR Addendums.

This is not intended to preclude issuing a new ExTR package on a more frequent basis, if the ExCB deems it appropriate. It is a recommended practice to consider issuing a new ExTR package anytime an extensive supplemental modification is being submitted.

# Receipt, Review and Audit of ExTR packages

The effective exchange of ExTR packages between Issuing and Receiving ExCBs is essential to the ability of manufacturers to obtain multi-country or multi-regional certifications based on a single test program and single submission of samples.

To accomplish this IECEx System objective, ExTR documents need to be developed and maintained in a timely and user-friendly fashion, and ExTR packages need to be compiled and approved by the Issuing ExCB with enough detail to clearly support all technical decisions.

If these actions are done effectively (from initial manufacturer request through final ExCB approval), such an approach can help facilitate the ability of a Receiving ExCB to review and audit an ExTR package without requiring re-testing or re-submittal of samples.

However, in accordance with IECEx 02, a Receiving ExCB may request a sample of the Ex equipment and copies of the documentation referred to in an ExTR Cover if such is deemed necessary. The receiving ExCB has the right to request an authenticated copy of the ExTR from the issuing ExCB. Authentication may be by affixing the stamp and official signature of the issuing ExCB to a copy of the ExTR or by other means acceptable to the receiving ExCB. See 4.2.3c) of IECEx OD 005-1, IECEx Quality System Requirements for Manufacturers – Part 1: Guidance on the establishment and maintenance of a quality system, and ISO/IEC 80079-35, Explosive atmospheres – Part 34: Application of quality systems for equipment manufacture, for additional quality details on the control of documents.

# Annex A – Example of Completed ExTR Cover (1 of 3)

| IECEx TEST REPORT COVER |  |  |  |
| --- | --- | --- | --- |
| ExTR Reference Number : |  | US/IC/ExTR11.00xx/00 |  |
| ExTR Free Reference Number : |  | US/IC 11NK12345 |  |
| Compiled by + signature (ExTL) : |  | Joe Compiler | Joe Compiler |
| Reviewed by + signature (ExTL) : |  | Jane Reviewer | Jane Reviewer |
| Approved by + signature (ExCB) : |  | Jim Approver | Jim Approver |
| Date of issue : |  | 2011-07-25 |  |
| Ex Testing Laboratory (ExTL) : |  | International Certification Inc. |  |
| Address : |  | 123 Street, New City, IL 12345 USA |  |
| Ex Certification Body (ExCB) : |  | International Certification Inc. |  |
| Address : |  | 123 Street, New City, IL 12345 USA |  |
| Applicant’s name : |  | International Instrumentation Company |  |
| Address : |  | Alter Strasse 123, D-12345 Neu-Stadt, Germany |  |
| Standards associated with this ExTR package : |  | IEC60079-0 Edition 5, IEC60079-11, Edition 5 |  |
| Clauses considered : |  | All clauses considered. |  |
| Test procedure : |  | IECEx System |  |
| Test Report Form Number : |  | ExTR Cover_4 (released 2010-12) |  |
| Test item description : | Mobile Phone |  |  |
| Model/type reference : | Type 1234-**** Type ABCD-**** |  |  |
| Code (e.g. Ex II T ) : | Ex ib IIC T4 - for the Type 1234-**** Ex ic IIC T4 - for the Type ABCD-**** |  |  |
| Rating : | 3.5 V battery, 2560 mAh |  |  |
| All testing fully performed by ExTL staff at ExTL address above: | No |  |  |
| Instructions for Intended Use of ExTR Cover: An ExTR Cover is the sole top-level document to associate together all other parts of an IECEx Test Report (ExTR) package. An ExTR package is comprised of an ExTR Cover and one or more associated ExTR documents (which may include Ex Test Reports, ExTR Addendums and ExTR of National Differences). All ExTR package documents are compiled and reviewed by the ExTL. The Issuing ExCB indicates final approval of the overall ExTR package on this ExTR Cover. |  |  |  |
| Copyright © 2011 International Electrotechnical Commission System for Certification to Standards Relating to Equipment for use in Explosive Atmospheres (IECEx System), Geneva, Switzerland. All rights reserved. This blank publication may be reproduced in whole or in part for non-commercial purposes as long as the IECEx System is acknowledged as copyright owner and source of the material. The IECEx system takes no responsibility for, and will not assume liability for, damages resulting from the reader's interpretation of the reproduced material due to its placement and context. |  |  |  |

# Annex A – Example of Completed ExTR Cover (2 of 3)

| Manufacturer’s name : | International Instrumentation Company |  |
| --- | --- | --- |
| Address : | Alter Strasse 123, D-12345 Neu-Stadt, Germany |  |
| Trademark : | IICo |  |
| ### Particulars: Test item vs. Test requirements |  |  |
| Classification of installation and use : |  | hand-held |
| Ingress protection : |  | IP20 |
| Rated ambient temperature range (°C) : |  | -20°C to +50°C |
| General remarks: The test results presented in this ExTR package relate only to the item or product tested. - "(see Attachment #)" refers to additional information appended to the ExTR package. - "(see appended table)" refers to a table appended to the ExTR package. - Throughout this ExTR package, a point “.” is used as the decimal separator. - Where the term “N/A” appears in any part of an ExTR package, it indicates that the associated issue was considered “Not applicable” to the involved evaluation. - In accordance with IECEx 02, a Receiving ExCB may request a sample of the Ex equipment and copies of the documentation referred to in an ExTR Cover. The technical content of this ExTR package shall not be reproduced except in full without the written approval of the Issuing ExCB and ExTL. |  |  |
| Copy of Marking Plate: N/A, Marking plate content not reviewed, as CoC not issued in conjunction with this ExTR |  |  |
| General product information: Mobile Phone, Type 1234-**** is a hand-held, intrinsically safe cellular telephone. Power is provided by a self-contained, rechargeable lithium-polymer battery cell. The variable nomenclature fields are for customer variants not affecting controlled construction. Model Phone, Type ABCD-**** is identical to the Type 1234-**** in construction, but is for Zone 2 locations. |  |  |
| In accordance with OD 024, testing not fully performed by ExTL staff at the above ExTL address: The following tests were performed by International Instrumentation Company staff at the above Applicant address and witnessed by International Certification Inc. ExTL staff in accordance with OD 024: - Test X - Test Y - Test Z The remainder of the tests were performed by International Certification Inc. ExTL staff at the above ExTL address. |  |  |
| National differences considered as part of this evaluation, if any: - This equipment also complies with the requirements of EN 60079-0:2006 and EN 60079-11:2007. - The differences between IEC 60079-0:2007, Edition 5 and EN 60079-0:2009, and between IEC 60079-11:2006, Edition 5 and EN 60079-11:2007, are covered in the IECEx Test Reports of National Differences. |  |  |
| “Conditions of Use” for Ex Equipment or “Schedule of Limitations” for Ex Components, if any: - Connection to the charging/communication port for charging and/or data transfer can only be made outside of an explosive atmosphere. The maximum safe area voltage for this port, Um, is 12V. - The battery assembly is only to be replaced by a trained technician outside of a hazardous location. Only IIC Type XYZ may be used. - Only IICo Type 7 and Type 8 SD cards may be used in the device. |  |  |
| Routine tests, if any: N/A |  |  |

# Annex A – Example of Completed ExTR Cover (3 of 3)

### Manufacturer’s Documents

| Title: | Drawing No.: | Rev. Level: | Date: |
| --- | --- | --- | --- |
| Marking | 1-ABCD | - | 2011-06-06 |
| Instruction Manual | 2-EFGH | 0 | 2011-04-24 |
| Overall Assembly Drawing | 3-IJKL | - | 2011-04-24 |
| Schematic | 4-MNOP | 04b | 2011-04-20 |
| Component and Circuit Trace Layouts | 5-QRST | 0 | 2011-04-24 |
| Parts List | 6-UVWX | 04b | 2011-04-20 |

# Annex B – Example of Completed Ex Test Report (1 of 3)

| IECEx TEST REPORT IEC 60079-0 Explosive atmospheres – Part 0: Equipment – General requirements |  |  |
| --- | --- | --- |
| ExTR Reference Number : | US/IC/ExTR11.00xx/00 |  |
| ExTR Free Reference Number : | US/IC 11NK12345 |  |
| Compiled by + signature (ExTL) : | Joe Compiler | Joe Compiler |
| Reviewed by + signature (ExTL) : | Jane Reviewer | Jane Reviewer |
| Date of issue : | 2011-07-25 |  |
| Ex Testing Laboratory (ExTL) : | International Certification Inc. |  |
| Address : | 123 Street, New City, IL 12345 USA |  |
| Applicant’s name : | International Instrumentation Company |  |
| Address : | Alter Strasse 123, D-12345 Neu-Stadt, Germany |  |
| Standard : | IEC 60079-0:2007, 5th Edition |  |
| Test procedure : | IECEx System |  |
| Test Report Form Number : | ExTR60079-0_5C (released 2010-08) |  |
| Instructions for Intended Use of Ex Test Report: An Ex Test Report provides a clause-by-clause documentation of the initial evaluation and testing that verified compliance of an item or product with an IEC Ex standard. This Ex Test Report is part of an ExTR package that may include other Ex Test Report, Addendum and National Differences documents, along with a single ExTR Cover. An Ex Test Report is to be compiled and reviewed by the ExTL. The Issuing ExCB indicates final approval of the Ex Test Report as part of the overall ExTR package on the associated ExTR Cover. |  |  |
| Copyright © 2011 International Electrotechnical Commission System for Certification to Standards Relating to Equipment for use in Explosive Atmospheres (IECEx System), Geneva, Switzerland. All rights reserved. This blank publication may be reproduced in whole or in part for non-commercial purposes as long as the IECEx System is acknowledged as copyright owner and source of the material. The IECEx system takes no responsibility for, and will not assume liability for, damages resulting from the reader's interpretation of the reproduced material due to its placement and context. |  |  |

| Possible test case verdicts: |  |
| --- | --- |
| - test case does not apply to the test item :N / A |  |
| - test item does meet the requirement …………………………..:Pass |  |
| General remarks: The test results presented in this Ex Test Report relate only to the item or product tested. - "(see Attachment #)" refers to additional information appended to this document. - "(see appended table)" refers to a table appended to this document. - Throughout this document, a point “.” is used as the decimal separator. The technical content of this Ex Test Report shall not be reproduced except in full without the written approval of the Issuing ExCB and ExTL. |  |

# Annex B – Example of Completed Ex Test Report (2 of 3)

| IEC 60079-0 |  |  |  |
| --- | --- | --- | --- |
| Clause | Requirement – Test | Result – Remark | Verdict |
| 1 | SCOPE |  |  |
| 2 | NORMATIVE REFERENCES |  |  |
| 3 | TERMS AND DEFINITIONS |  |  |
| 4 | Apparatus grouping and temperature classification |  |  |
| 4.1 | Group I | The device has not been evaluated for Group I. | N/A |
| 4.2 | Group II | The device is for Group IIC. | Pass |
| 4.3 | Group III | The device has not been evaluated for Group III. | N/A |
| 4.4 | Equipment for a particular explosive atmosphere | The device has been evaluated for Group IIC. | N/A |
| 5 | Temperatures |  |  |
| 5.1 | Environmental influences |  | Pass |
| 5.1.1 | Ambient temperatures | The device is intended for an ambient temperature range of –20°C to +50°C. The marking “-20°C ≤ Tamb ≤ +50°C” is provided on the marking label. | Pass |
| 5.1.2 | External source of heating or cooling | The device is hand-held and not intended to be connected to a separate source of heating or cooling. | N/A |
| 5.2 | Service temperature | Since the device is a hand-held, battery-powered device, the service temperature was considered to be the same as the maximum ambient temperature. | Pass |
| 5.3 | Maximum surface temperature |  | Pass |
| 5.3.1 | Determination of maximum surface temperature | The device was assessed for temperature rise in accordance with IEC 60079-11:2006. | Pass |
| 5.3.2 | Limitation of maximum surface temperature | The device was assessed for temperature rise in accordance with IEC 60079-11:2006. | Pass |
| 5.3.2.1 | Group I electrical equipment | The device was not evaluated for Group I. | N/A |
| 5.3.2.2 | Group II electrical equipment | The device was assigned a temperature class of T4 based on evaluation to IEC 60079-11:2006. See the ExTR for IEC 60079-11:2006 for details. | Pass |
| 5.3.2.3 | Group III electrical equipment |  |  |

Annex B – Example of Completed Ex Test Report (3 of 3)

| IEC 60079-0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Clause | Requirement – Test | Result – Remark |  |  | Verdict |
| 5.3.2.3.1 | Maximum surface temperature determined without a dust layer | The device was not evaluated for Group III. |  |  | N/A |
| 5.3.2.3.2 | Maximum surface temperature with respect to dust layers | The device was not evaluated for Group III. |  |  | N/A |
| 5.3.3 | Small component temperature for Group I or Group II electrical equipment | Small components used in the device were assigned a temperature class of T4 based on evaluation to IEC 60079-11:2006. See the ExTR for IEC 60079-11:2006 for details. |  |  | Pass |
| 6 | REQUIREMENTS FOR ALL ELECTRICAL EQUIPMENT |  |  |  |  |
| 6.1 | General | The device has been evaluated to IEC 60079-0:2009 and IEC 60079-11:2006. |  |  | Pass |
| 6.2 | Mechanical strength of equipment | The device was subjected to a drop test in accordance with 26.4.3. |  |  | Pass |
| 6.3 | Opening times | Excluded per Table 1 in IEC 60079-11:2006. |  |  | N/A |
| 6.4 | Circulating currents | The device is hand-held. |  |  | N/A |
| 6.5 | Gasket retention | Gaskets are formed by an over-mold material bonded to the main housing. |  |  | Pass |
| . . . |  |  |  |  |  |
| 30 | Instructions |  |  |  |  |
| 30.1 | General | The instructions contain the necessary information. |  |  | Pass |
| 30.2 | Cells and batteries |  | The instructions contain battery replacement instructions and warnings. |  | Pass |
| Annex A (Normative) | SUPPLEMENTARY REQUIREMENTS FOR cable glands |  |  | Excluded per Table 1 in IEC 60079-11:2006. | N/A |
| Annex B (Normative) | Table B.1 – Clauses with which Ex components shall comply |  |  |  | N/A |

Measurement Section, including Additional Narrative Remarks (as deemed applicable) N/A

# Annex C – Example of Completed ExTR of National Differences (1 of 2)

| IECEx TEST REPORT of NATIONAL DIFFERENCES |  |  |
| --- | --- | --- |
| ExTR Reference Number : | US/IC/ExTR11.00xx/00 |  |
| ExTR Free Reference Number : | US/IC 11NK12345 |  |
| Compiled by + signature (ExTL) : | Joe Compiler | Joe Compiler |
| Reviewed by + signature (ExTL) : | Jane Reviewer | Jane Reviewer |
| Date of issue : | 2011-07-25 |  |
| Ex Testing Laboratory (ExTL) : | International Certification Inc. |  |
| Address : | 123 Street, New City, IL 12345 USA |  |
| Applicant’s name : | International Instrumentation Company |  |
| Address : | Alter Strasse 123, D-12345 Neu-Stadt, Germany |  |
| Country/Region : | Europe: Switzerland (CH), Czech Republic (CZ), Germany (DE), Denmark (DK), Finland (FI), France (FR), United Kingdom (GB), Hungary (HU), Italy (IT), the Netherlands (NL), Norway (NO), Romania (RO), Sweden (SE) and Slovenia (Sl) |  |
| Standards : | IEC 60079-0, Fifth Edition |  |
| Test procedure : | IECEx System |  |
| Test Report Form Number : | ExTR National Differences_2 (released 2010-08) |  |
| Instructions for Intended Use of ExTR of National Differences: An ExTR of National Differences is to supplement an ExTR or ExTR Addendum, with a separate such document issued for each intended country / region. All National Differences of the intended country / region are to be tabulated and remarked upon as part of this document, even those that are not applicable. An ExTR of National Differences is to be compiled and reviewed by the ExTL. The Issuing ExCB indicates final approval of the ExTR of National Differences as part of the overall ExTR package on the associated ExTR Cover. |  |  |
| Copyright © 2011 International Electrotechnical Commission System for Certification to Standards Relating to Equipment for use in Explosive Atmospheres (IECEx System), Geneva, Switzerland. All rights reserved. This blank publication may be reproduced in whole or in part for non-commercial purposes as long as the IECEx System is acknowledged as copyright owner and source of the material. The IECEx system takes no responsibility for, and will not assume liability for, damages resulting from the reader's interpretation of the reproduced material due to its placement and context. |  |  |

| Possible test case verdicts: |  |
| --- | --- |
| - test case does not apply to the test item :N / A |  |
| - test item does meet the requirement …………………………..:Pass |  |
| General remarks: The test results presented in this ExTR of National Differences relate only to the item or product tested. Only clauses and manufacturer’s documents impacted by this document are detailed. - "(see Attachment #)" refers to additional information appended to this document. - "(see appended table)" refers to a table appended to this document. - Throughout this document, a point is used as the decimal separator. The technical content of this ExTR of National Differences shall not be reproduced except in full without the written approval of the Issuing ExCB and ExTL. |  |

# Annex C – Example of Completed ExTR of National Differences (2 of 2)

| EN 60079-0:2009 |  |  |  |
| --- | --- | --- | --- |
| Clause | Requirement – Test | Result – Remark | Verdict |
| Annex ZA (Normative) | Normative references to international publications with their corresponding European publications | Considered as applicable. | Pass |
| Annex ZY | Equipment groups and marking examples |  |  |
| Annex ZY | Equipment groups | The devices are for Group IIC, Categories 1 and 2. | Pass |
| Annex ZY | Instructions | Provided as such. | Pass |
| Annex ZY | Marking | Provided as such. | Pass |
| Annex ZZ | Additional information in EN 60079-0 regarding the coverage of Essential Requirements of EC Directives. |  |  |
| Annex ZZ | ESR 1.3.5 | The device does not contain pressure. | N/A |
| Annex ZZ | ESR 1.5.1 | The device does not perform a safety related function with resepct to explosive atmospheres. | N/A |
| Annex ZZ | ESR 1.5.2 | The device does not perform a safety related function with resepct to explosive atmospheres. | N/A |
| Annex ZZ | ESR 1.5.3 | The device does not perform a safety related function with resepct to explosive atmospheres. | N/A |
| Annex ZZ | ESR 1.5.4 | The device does not perform a safety related function with resepct to explosive atmospheres. | N/A |
| Annex ZZ | ESR 1.5.5 | The device does not perform a measuring function for explosion protection. | N/A |
| Annex ZZ | ESR 1.5.6 | The device does not perform a measuring function for explosion protection. | N/A |
| Annex ZZ | ESR 1.5.7 | The device does not perform a measuring function for explosion protection. | N/A |
| Annex ZZ | ESR 1.5.8 | The device does not perform a safety related function with resepct to explosive atmospheres. | N/A |
| Annex ZZ | ESR 1.6.1 | The device is battery powered. | N/A |
| Annex ZZ | ESR 1.6.3 | The device is battery powered. | N/A |
| Annex ZZ | ESR 1.6.5 | The device does not monitor explosive atmospheres. | N/A |
| Annex ZZ | ESR 3.0.1 | The device is not a protective system. | N/A |

.

.

.

| Annex ZZ | ESR 3.1.7 | The device is not a protective system. | N/A |
| --- | --- | --- | --- |
| Annex ZZ | ESR 3.1.8 | The device is not a protective system. | N/A |

Measurement Section, including Additional Narrative Remarks (as deemed applicable) N/A

# Annex D – Example of Completed ExTR Addendum (1 of 2)

| IECEx TEST REPORT ADDENDUM |  |  |
| --- | --- | --- |
| ExTR Reference Number : | US/IC/ExTR11.00xx/00 |  |
| ExTR Free Reference Number : | US/IC 11NK12345 |  |
| Compiled by + signature (ExTL) : | Joe Compiler | Joe Compiler |
| Reviewed by + signature (ExTL) : | Jane Reviewer | Jane Reviewer |
| Date of issue : | 2011-07-25 |  |
| Ex Testing Laboratory (ExTL) : | International Certification Inc. |  |
| Address : | 123 Street, New City, IL 12345 USA |  |
| Applicant’s name : | International Instrumentation Company |  |
| Address : | Alter Strasse 123, D-12345 Neu-Stadt, Germany |  |
| Standards : | IEC 60079-0 Edition 4 |  |
| Test procedure : | IECEx System |  |
| Test Report Form Number : | ExTR Addendum_2 (released 2010-08) |  |
| Instructions for Intended Use of ExTR Addendum: An ExTR Addendum is to supplement a previously issued ExTR package. Only those clauses applicable to the supplemental issue being addressed are to be tabulated and remarked upon as part of this document. An ExTR of National Differences may also supplement this document. An ExTR Addendum is to be compiled and reviewed by the ExTL. The Issuing ExCB indicates final approval of the ExTR Addendum as part of the overall ExTR package on the associated ExTR Cover. |  |  |
| Copyright © 2011 International Electrotechnical Commission System for Certification to Standards Relating to Equipment for use in Explosive Atmospheres (IECEx System), Geneva, Switzerland. All rights reserved. This blank publication may be reproduced in whole or in part for non-commercial purposes as long as the IECEx System is acknowledged as copyright owner and source of the material. The IECEx system takes no responsibility for, and will not assume liability for, damages resulting from the reader's interpretation of the reproduced material due to its placement and context. |  |  |

| Possible test case verdicts: |  |
| --- | --- |
| - test case does not apply to the test item :N / A |  |
| - test item does meet the requirement …………………………..:Pass |  |
| General remarks: The test results presented in this ExTR Addendum relate only to the item or product tested, and are only valid when considered together with the related Ex Test Report that was previously issued, along with any previously issued ExTR Addendums for the same item or product. Only clauses and manufacturer’s documents impacted by this document are detailed. - "(see Attachment #)" refers to additional information appended to this document. - "(see appended table)" refers to a table appended to this document. - Throughout this document, a point “.” is used as the decimal separator. The technical content of this ExTR Addendum shall not be reproduced except in full without the written approval of the Issuing ExCB and ExTL. |  |

# Annex D – Example of Completed ExTR Addendum (2 of 2)

| Clause | Requirement – Test | Result – Remark | Verdict |
| --- | --- | --- | --- |
| 5 | Temperatures |  |  |
| 5.1 | Environmental influences |  |  |
| 5.1.1 | Ambient temperature | High voltage flameproof enclosures are for use in an ambient temperature range of -20C to +55C. This ambient temperature range is marked on the enclosures. | Pass |
| 5.2 | Service temperature | Service temperature is determined at the maximum ambient of +55C and maximum rated conditions for the enclosures. Conductor size on B series is being decreased. Temperature tests previously conducted on A series were considered representative of this change. Conductor size changes on Y series were considered covered by temperature tests previously conducted on X series. | Pass |
| 5.3 | Maximum surface temperature |  |  |
| 5.3.1 | Determination of maximum surface temperature | Maximum surface temperature was determined by the voltage chosen per Clause 26.5.1 at the maximum ambient of +55C. Conductor size on B series is being decreased. Temperature tests previously conducted on A series were considered representative of this change. Conductor size changes on Y series were considered covered by temperature tests previously conducted on X series. | Pass |
| 5.3.2 | Limitation of maximum surface temperature |  |  |
| 5.3.2.2 | Group II electrical apparatus | High voltage flameproof enclosures are marked with a temperature class in accordance with Table 2. Enclosures for use in an ambient temperature range of -20C to +55C are marked with a T6 temperature code. | Pass |
| 5.4 | Surface temperature and ignition temperature | The maximum surface temperature does not exceed the temperature class. | Pass |

Measurement Section, including Additional Narrative Remarks (as deemed applicable) N/A