- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE: Draft ExTAG Decision Sheet – Vibration test on partition walls

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This ExTAG Draft Decision Sheet was prepared by SGS BASEEFA and is issued for a 4 week comment period.

Please submit comments using the attached comments table by -

2015 04 14 to

Christine Kane

If no comments are received the document will proceed for publication.

On behalf of Mr. Gauthier

- Julien Gauthier

Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

| Member Body | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation (to be completed by the originator) |
| --- | --- | --- | --- | --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISION

| Standard: IEC 60079-26:2014 | Clause: 5.2 | - Date: 2015-03-10 Draft Originator of proposal: SGS Baseefa |
| --- | --- | --- |
| Subject: Vibration test on partition walls Status of document: Draft | Key words: - Vibration test - Partition wall - Separation element | TC/SC involved: MT 60079-26 |
| Question: Clause 5.2 of IEC 60079-26:2014 introduces two tests for Partition Walls in accordance with 4.1.3. The first test is a thermal endurance test followed by a pressure test. This should not pose a problem for any laboratory with the pressure testing equipment required for IEC 60079-1. In the event that the required pressure is in excess of the laboratory’s capability, the test should be capable of being performed at the manufacturer’s location under an OD 024 agreement. The second test applies only to partition walls subject to constant vibrational stress, but the stated test appears to be a whole life test, rather than an accelerated test, using the words: the specified vibrational endurance limit (cycles) shall be verified at the maximum amplitude and at the specified temperature limits. Is this intended to be a whole life test, which would normally be impractical to be performed by an ExTL, or can an alternative accelerated test be substituted? If so, what? Draft Answer: It is not practical for an ExTL to perform a whole life test. Either the ExTL must satisfy themselves that existing whole life data provided by the manufacturer is reliable, or they must agree an accelerated test procedure with the manufacturer. |  |  |