- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE: Draft ExTAG Decision Sheet – Flameproof joint for enclosure lid of non-cylindrical shape

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This ExTAG Draft Decision Sheet was prepared by INERIS to assist in applying a consistent approach when assessing flameproof joint for enclosure lid of non-cylindrical shape.

Please submit comments using the attached comments table by -

2015 01 31 to

Christine Kane

If no comments are received the document will proceed for publication.

On behalf of Mr. Gauthier

- Julien Gauthier

Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

| Member Body | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation |
| --- | --- | --- | --- | --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISION

| Standard: IEC 60079-1:2014 (Edition 7.0) IEC 60079-1:2007 (Edition 6.0) | Clause: 5.2 & 5.2.3 5.2 & 5.2.3 |  |
| --- | --- | --- |
| Subject: Spigot joints/Conical joint | Key words: - Flameproof joints - Spigot joint - Conical joint - Flanged joint | Date: 2014-12-12 Originator of proposal: INERIS TC/SC involved: |
| Introduction : A manufacturer need to design a flameproof enclosure with a geometry as in the example of Figure 1. This enclosure could be manufactured with a joint either following the principle of the Figure 2 or following the principle of the Figure 3. Figure 1 : Example of flameproof enclosure Figure 2 : Perpendicular assembly between the cover and the body ≤5° taper Figure 3 : Conical assembly between the cover and the body Background: - A similar question has been assessed by the European ATEX Notified Bodies Group. The answer have been formalized in Clarification Sheet n°ExNB/00/031/CS with the following conclusion : “Extract of Clarification Sheet n°ExNB/00/031/CS” - According to the UL 1203 standard, a spigot (rabbet) joint consists of an axial segment and a radial segment which are arranged to be at right angles to each other. The actual shape of the enclosure parts is not considered, but only the right angle relationship of the segments. This is the same relationship as is shown in Figures 2 and 3 above. The construction must meet minimum lengths of the segments of the joint and maximum gaps, including acetylene atmospheres. Testing compliance must be achieved. Similarly, IEC 60079-1:2014 construction and performance requirements are applied: - the cross-section of this type of joint is reflected by the Figure 2a or 2b of width and gap of the flameproof joint in accordance with Table 3 as spigot joint, - Testing compliance must be achieved Question: How these types of flameproof joints (Figure 2and Figure 3 shown above) could be considered according to 5.2 Non-threaded joints clause of IEC 60079-1 7th edition in order to be used for all the gas Group (I, IIA, IIB and IIC)? Answer: These joints can be considered as spigot joint according clause 5.2.3 of IEC 60079-1, provided that: - in case of Figure 2 - The minimum values of width of joint and maximum gaps are in accordance with the Table 2 or 3 of IEC 60079-1:2014 standard (equivalent to Table 1 or 2 of IEC 60079-1:2007). 2) The equipment passed successfully the tests in accordance with 15.3.3 of the IEC 60079-1:2014 standard (equivalent to 15.2.2 of IEC 60079-1:2007). 3) The values of the width of joint and the gaps shall be measured accurately for the tests and the production control (using e.g. a Coordinate Measuring Machines (CMMs) equipment); NOTE: CMM measurement uncertainty shall be suitable in comparison to the gap absolute measurement (e.g. a safe gap 40 µm all along the path according geometry) considering that the use of a CMM instrument with good performance does not imply to obtain good measurement results; it is because that the reliability of measurement results is not only attributed to the instrument, but also the environment, operator, and method are involved; therefore is mandatory to determine a measurement uncertainty to characterize the reliability of measurement results in order to guarantee an expected accuracy. 4) 100% of measurement is mandatory and sampling in production is not permitted. The ExCB involved in QSM according ISO 80079-34 shall perform the audit considering the measurement process, as described above in 3), in particular when evaluating the acceptance criteria in 16.5 of IEC 60079-1:2014 (equivalent to 16.3 of IEC 60079-1:2007), in consideration of the fact that the enclosure shall withstands the pressure without suffering permanent deformation of the joints, which means the measurement before and after the pressure test in order to evaluate a deformation. - in case of Figure 3 as well as in case a), provided the same conditions above per Figure 3 and ensuring that the cone angle do not exceed 5° through the production measurement control as per a) 3) and 4). |  |  |