INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE

IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulated to: ExTAG – IECEx Testing and Assessment

- TITLE: Draft Decision Sheet - Group Verification Requirements for Flameproof Castings

INTRODUCTION

- This ExTAG Draft Decision Sheet, ExTAG/256/CD Draft Decision Sheet -. Group Verification Requirements for Flameproof Castings has been prepared by UL.

It is now issued for a 4 week comment period.

Please submit comments using the attached Comments Table by-

2012 05 02 to

Christine Kane

If comments have not been received by this date the document will proceed for publication.

On behalf of Mr. Brenon

## Michel Brenon

ExTAG Secretary

| Address: IECEx Secretariat 286 Sussex Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 E-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

| Member Body/ Country | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation |
| --- | --- | --- | --- | --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: ISO/IEC 80079-34 | Clauses: A.3.2 |  |
| --- | --- | --- |
| Subject: Verification Requirements for Flameproof Castings Status of document: Draft | Key words: - Flameproof enclosures - Verification - Castings - 100% visual inspection | # Date: Originator of proposal: UL TC/SC involved: IEC/TC 31 and IEC/SC 31M |
| Question: Does A.3.2 require 100% visual inspection of castings to verify wall thickness, flaws, inclusions, blow holes and porosity? Answer: No. Annex A is Informative, not Normative. Therefore, it merely provides guidance regarding issues to consider. Therefore, when addressing Product realization, such as under 7.1 and 7.5.1, control of the casting process is needed regarding wall thickness, flaws, inclusions, blow holes and porosity. This control can be accomplished by 100% visual inspection, or by another means deemed appropriate based on the ability of the manufacturer to effectively control production. |  |  |