### INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM

FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR

USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE: Draft ExTAG Decision Sheet – What Standards must be listed on page 2 of the IECEx Certificate?

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This updated ExTAG Draft Decision Sheet ExTAG/243B/CD ExTAG Draft Decision Sheet – What Standards must be listed on page 2 of the IECEx Certificate? has been prepared by TestSafe Australia taking into account comments received on ExTAG/243A/CD, contained in ExTAG/263/CC - Compilation of comments on ExTAG/243A/CD Draft ExTAG Decision Sheet – What Standards must be listed on page 2 of the IECEx Certificate?

This document has been redrafted to include an Option a) and Option b) under Question 1and has been issued for discussion during the Calgary 2012 ExTAG Meeting.

On behalf of Mr. Brenon

## Michel Brenon

ExTAG Secretary

| Address: IECEx Secretariat 286 Sussex Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 E-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

IECEx / ExTAG DECISION SHEET

Title: ExTAG Decision Sheet – What Standards must be listed on page 2 of the IECEx Certificate?

| Standard: #### All | Clause: | Sheet No. |
| --- | --- | --- |
| Subject: What Standards must be listed on page 2 of the IECEx Certificate? | Key words: - Certificate - Standards - Revision Numbers - Issue - Marking | # Date: 2011-11-14 (original) Originator of proposal: TestSafe Australia TC/SC involved: Nil |
| Related documents: | Nil |  |

Introduction:

There are some differences in opinion on how to prepare certificates where the equipment (call it Ex d Gate End Box) may be made up of several separately certified Ex equipment or components.

The separately certified Ex equipment or components may have been issued with IECEx certificates to different types of protection and Standards (e.g. Ex m, Ex i, Ex e etc) and the Standards may have different issue numbers as well (one Ex i level transducer may have used IEC 60079-0:2011 with IEC 60079-11:2011, another Ex i level transducer may have been certified some time back and may have used IEC 60079-0:2004 with IEC 60079-11:2006. An Ex m encapsulated coil may have used IEC 60079-0:2007 with IEC 60079-18:2009).

When preparing the report and certificate for the complete equipment (Gate End Box) this is predominantly an Ex d box. Inside the equipment description we would be describing all the Ex equipment and components that make up the complete Gate End Box, with all the Ex certificates that have been used for these separately certified equipment.

Question 1:  When we prepare the certificate for the complete Gate End Box, what Standards do we list on page 2 of the certificate?

Answer 1 Option a): It is sufficient to only list the major Standards IEC 60079-0:2011 and IEC 60079-1:2007 applicable on the page 2 of the IECEx certificate.  It is not necessary to list all the different Standards that were used by the separately certified equipment that make up the Gate End Box.

But the complete details of the certificates for the separately certified equipment and components shall be provided in the equipment description.

Answer 1 Option b): All the Standards applicable, including Edition (or year of issue) shall be listed on the page 2 of the IECEx certificate. Where the various parts of the equipment use different certificates, the IEC Standards on page 2 of the CoC shall list  the ‘oldest’ edition from all the applicable certificates (for example, if Ex d certificate is to IEC 60079-0:2011, but Ex e certificate is to IEC 60079-0:2007, the CoC will list IEC 60079-0:2007), unless a gap analysis has been conducted and reported to determine if the product still meets the requirements of the standards being stated in the IECEx Certificate.

In addition, complete details of the certificates for the separately certified equipment and components shall be provided in the equipment description.

Question 2: What types of protection should be listed in the marking code?

Answer 2 Option a): The marking code must list the major type of protection (Ex d I). If practical, it may include the other types of protection applicable too.

Answer 2 Option b): The marking code must list all the types of protection used by the equipment and its parts, with the symbols for the various types of protection to appear in alphabetical order, as provided in IEC 60079-0:2011 Clause 29.6.

Question 3: Which IECEx TR and QAR should be listed in the certificate for the overall equipment?

Answer 3 Option a):  The certificate must list the IECEx TR and QAR for the major type of protection for the overall equipment. It need not list the IECEx TR’s and QAR’s relevant to the separately certified equipment or components.

Answer 3 Option b):  The certificate must list all the IECEx TR’s and QAR’s relevant to the separately certified equipment or components.