INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE

IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulated to: ExTAG – IECEx Testing and Assessment Group

TITLE: Draft Decision Sheet - Spigot joints

INTRODUCTION

This proposed Draft ExTAG Decision Sheet was prepared by TestSafe, AU, and is issued for your consideration.

Please submit comments by -

1st March 2010 to ExTAG Secretary, Mr. Michel Brenon, via

Christine Kane

If comments have not been received by the above date this document will be published as an ExTAG Decision Sheet.

## Michel Brenon

ExTAG Secretary

| Address: IECEx Secretariat 286 Sussex Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 E-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

| Member Body | Clause/ Subclause | Paragraph Figure/ Table | Type of comment General/technical/ editorial | COMMENTS | Proposed change | Observation |
| --- | --- | --- | --- | --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: IEC 60079-1:2007 (Ed 5) | Clauses: 5.2.3 |  |
| --- | --- | --- |
| Subject: # Spigot joint Status of document: Draft | Key words: - Spigot joint - Rectangular/oblong | # Date: 100118 Originator of proposal: TSA TC/SC involved: |
| Question: Can a flameproof joint consist of a spigot joint made up of a rectangular shape, with or without rounded corners? Also, would an oblong joint comply? (Explanation of question: Normally spigot joints consist of a cylindrical part with a further plane part. In this case, the cylindrical part is made up of a rectangular section and also a rounded section at the corners). Answer: A previous ATEX Clarification/Decision Sheet number ExNB/00/031/CS issued 2004/03/09 had provided a decision that such rectangular joints with rounded corners were acceptable without any limitation for Groups I, IIA, IIB. Restrictions were provided when considering Group IIC. Computerised machining methods now provide consistent machining which comply to a high degree with the drawing data input into such machining centres. Therefore, for this decision sheet under IECEx, it is proposed that such joints, including oblong joints, are allowed for Groups I, IIA, IIB, provided the flameproof joint width and gaps comply with Table 1 of IEC 60079-1:2007. Special methods will be required to measure the gaps, which may consist of making special gauges using computerised machining. |  |  |