INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulated to: ExTAG – IECEx Testing and Assessment Group

- TITLE: Compilation of comments on ExTAG/307/CD - Draft Decision Sheet – Evaluation of stator windings & insulation system

INTRODUCTION

This document is a compilation of comments received, as well as observations from the originator Mr. John Allen, on behalf of TC 31 WG 27, on ExTAG/307/CD, Draft Decision Sheet – Evaluation of stator windings & insulation system

A revised document has been prepared taking into account these comments and observations, ExTAG/307A/CD - Draft Decision Sheet – Evaluation of stator windings & insulation system, which has been circulated for a further one month comment period.

On behalf of Mr. Brenon

- Michel Brenon

ExTAG Secretary

| Address: IECEx Secretariat 286 Sussex Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 E-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

| Member Body/ Country | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation |
| --- | --- | --- | --- | --- | --- | --- |
| TC31 Chair | Question |  |  | The question is more specific than the answer | Question: What information is required from the OEM stator winding to ensure that the machine with a copy rewind will behave with similar electro-mechanical and thermal performance as the OEM machine plus what else needs to be done to be confident the machine will behave in the same manner as the original machine? | Accept in principle Proposed amendment “What information is required from the OEM stator winding and what additional process requirements and controls are necessary to provide confidence that a copy rewound machine will behave in the same manner as the original machine” |
| DEKRA NL | Page 2 | 5th paragraph | General / technical | " Impregnation Process:- The method of impregnation used by the OEM should be evaluated identifying if the winding had been dip, VPI or trickle impregnated. The copy winding should have the same process and if that is not possible have an alternative process to achieve the same penetration and build of impregnating resin onto the winding. " All options of this requirement are stated with a “should”. This means, there is no mandatory decision in this requirement. Consequently it becomes an informative note. | Change the second sentence to: " The copy winding should have the same process and if that is not possible the copy winding shall have an alternative process to achieve the same penetration and build of impregnating resin onto the winding. " | Accept |
| DNV NEMKO NO |  |  |  | No comments |  |  |
| Ex-Agencija HR |  |  |  | Ex-Agencija support the document |  |  |
| FM GB |  |  |  | This is a repair issue and we have no comments on this subject. |  |  |
| FM US |  |  |  | Same situation for FM Approvals LLC (FMG) |  |  |
| FTZU CZ |  |  |  | We do not have any comments to the ExTAG/307/CD document. |  |  |
| NANIO CCVE RU |  |  | General | We support ExTAG/307/CD without any comments. |  |  |
|  |  | Answer, paragraph 7, the last sentence | Editorial | There is a misprint in the sentence: The number of coils in each coil group and the sequence of these coil groups within each phase and around the stator bore must be recorded. | Replace bore to core: The number of coils in each coil group and the sequence of these coil groups within each phase and around the stator core must be recorded. | Accept |
|  |  | Answer, paragraph 14, the last Sentence | Editorial | There is a misprint in the sentence: The strength of the binding materials shall be equivalent of better than those used by the OEM | Replace of to or: The strength of the binding materials shall be equivalent or better than those used by the OEM | Accept |
| NEPSI CN |  |  |  | On behalf of NEPSI, I agree to the proposal. |  |  |
| SIRA GB | Ans: Paras 10, 12, 14, 15 |  | Ed | Type errors | Minor editorial changes, see amendments in submission |  |
|  | Ans: para 13 |  | Tech | The power output of the anti condensation heater should be considered | Add “wattage” to the text: …anti-condensation heaters must be replaced by similar heaters with the same design type, wattage, and thermal rating… | Accept |
|  | Ans: para 15 |  | Tech | Additional information required regarding the number of layers or number of processes. | Impregnation Process:- The method of impregnation used by the OEM should be evaluated identifying if the winding had been dipped, VPI or trickle impregnated, and the number of layers/processes. The copy winding should have the same process and if that is not possible have an alternative process to achieve the same penetration and build of impregnating resin onto the winding. | Reject It is not practicable for the service facility to determine the number of layers or processes the OEM used or to differentiate from additional layers added during overhauls in service |
| UL/USA |  | 1st paragraph | General | In the statement: “For Ex e no variation in winding form, other protection concepts can have electro-mechanically equivalent windings”, what is the definition of electro-mechanically equivalent? Does a motor need to meet each item in the following list in order to be considered Electro-mechanically equivalent? | Clarify the term “electro-mechanically equivalent” | Accept The winding shall have the same number of effective conductors per slot although the winding type may vary. Eg from concentric to lap This needs to take into account the pitch and distribution factors of the original and amended windings |
|  |  | All | General | In the first paragraph, a distinction is made between ‘e’ motors and other methods of protection. Does the entirety of the following list apply to ‘e’ only, or does it apply to all protection types (‘d’, ‘p’, etc.)? | Clarify the applicability of the list of critical features for each protection type. | Accept in principle The entirety of the following list applies to all copy rewinds irrespective of protection type There is an additional requirement for Ex e which is that the winding type or form shall not be changed |