## COLLECTION OF IECEx/ExTAG DECISIONS

| Subject: Use of IECEx Certified Components Status of document: Approved | Key words: - procedures - manufacturers - components | ### Decision of meeting Date: 3 November 2005 Originator of Proposal: Chairman Panel of Assessors |
| --- | --- | --- |

Question:   Is it possible to certify an item of Ex equipment where the equipment includes unspecified IECEx certified components?

Answer:

With the exception of cable glands, the answer to this is NO, but there are opportunities to provide certificate holders with flexibility in the use of components.

Certifying equipment with general reference to the use of IECEx certified components could lead to a number of potential problems. These may include:

- Due to lack of certified components that equipment could not actually be sold or used as a certified.

- Again due to lack of certified components, and the potential confusion of the manufacturers, they could be selling products that they mistakenly assume to be certified but contains uncertified components.

- The components used may of the wrong rating and compromise compliance or safety.

- The components could be of the wrong size, and compromise compliance for such things as creepage and clearance distances.

Any item of application for testing under the IECEx Scheme must clearly identify any components that contribute to the explosion protection of the apparatus. Where these are from a different manufacturer to the main apparatus, model/component number, rating, critical physical dimensions etc must be identified by the manufacturer.

These identified components may be tested as part of the process or, if they have IECEx component certificate in their own right, they may be referenced in the ExTR, The use of these components in the finally assembly should be checked both in the drawings and on the assembled equipment.  All information should be clearly documented in the ExTR.

In addition to the above, manufacturers are permitted to reference ALTERNATIVE IECEx certified components, provided the relevant parameters such as rating and size are clearly specified.  Assessment of the potential suitability of the relevant parameters for such alternative components based on the parameters specified should also be addressed in the ExTR.

See Decision Sheet DS2004/006 for more information on the topic of use components not covered by an ExTR.

Although not covered by an ExTAG decision sheet, ExTAG has previously addressed the use of certified glands.  The applicant is not obliged to provide a sample certified gland with the equipment unless it is integral part of the design of the equipment or it will needed as part of the test program.

In summary, each end product incorporating the components must be subject to full evaluation, with the information relating to the evaluation of each component being presented to and examined by the body evaluating the end product. This principle should be applied to all Ex components with the exception of cable glands.

--------------------------