| Reference No: |  |  |  |  |  |  |  |  | #### IEC 60079-7 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Field of application This form is intended to be used when assessing and testing electrical apparatus for explosive atmospheres according to the standard specified in the adjoining column. The test results are valid for the tested items only. Reproduction This report must not be reproduced other than in its entirety except with the prior written approval of | Standard: # IEC 60079-0 : 2004 # Edition 4 Electrical apparatus for explosive gas atmospheres – General requirements AND # IEC 60079-7: 2001 Third edition Electrical apparatus for explosive gas atmospheres – Part 7: Increased safety "e" |  |  |  |  |  | ## Instructions for use The comment columns can be used to e.g. specify test values, applied alternative requirements/tests or to refer to enclosed additional pages with comments. Abbreviations: N/A = Not applicable (for the assessment/test of the equipment and its design documentation) The applicant is the body/person who has requested the assessment/test and to which the results, as specified in this report, are provided. The short descriptions in the column for "Clause" do not replace the complete text in the standard. |  |  |  |  |
| KEMA Quality B.V. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | # This test report may only be used for the # IECEx Scheme and must be accompanied by an ExTR cover page issued by an ExCB. |  |  |  |  |
| ## Product name |  |  |  |  |  | # Model/Type designation |  |  |  |  |  |
| Serial No |  | Code (e.g. Ex II T ) |  |  |  |  |  | Ambient temperature [min-max] |  |  |  |
| Pre-production sample |  |  |  |  |  |  |  |  |  |  |  |
| Applicant [name and postal address] |  |  |  |  |  | Manufactured by [name and postal address] |  |  |  |  |  |
| Test sample received [date] |  |  |  |  |  |  |  |  |  |  |  |
| ## Product description |  |  |  |  |  |  |  |  |  |  |  |
| The equipment complies with the standard [yes/no] |  | ## Prepared by |  |  |  |  |  |  |  |  | Date [year-month-day] |
|  |  | [signature] |  |  |  |  |  |  |  |  |  |

| Reference No: |  |  |  |  |  | IEC 60079-0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  | Results |  |  |  |  |  |
|  |  | Pass | Fail | N/A | Comments |  | ExCB Review |
| 1 | Scope | x |  |  |  |  |  |
| 2 | Normative references | x |  |  |  |  |  |
| 3 | Terms and definitions | x |  |  |  |  |  |
| 4 | Apparatus grouping and temperature classification | x |  |  |  |  |  |
| 5.1.1 | Ambient temperatures | x |  |  |  |  |  |
| 5.1.2 | External source of heating or cooling |  |  | x |  |  |  |
| 5.2 | Service temperature See also DS 2015/011A | x |  |  | Operating temperature range ! |  |  |
| 5.3 | Maximum surface temperature See also DS 2015/011A | x |  |  |  |  |  |
| 5.5 | Small component temperature |  |  | x |  |  |  |
| 6 | Requirements for all apparatus |  |  |  | only apply to 6.1 |  |  |
| 13 | Ex components | x |  |  |  |  |  |
| 24 | Documentation |  |  |  |  |  |  |
| 25 | Compliance of prototype or sample with documents |  |  |  |  |  |  |
| 26.1 | Type tests, General |  |  |  |  |  |  |
| 26.5.1 | Temperature measurement |  |  |  | according EN 60947-7-1:1991 Clause 8.3.3 |  |  |
| 26.7.2 | Temperatures during tests |  |  |  |  |  |  |
| 26.8 | Thermal endurance to heat |  |  |  |  |  |  |
| 26.9 | Thermal endurance to cold |  |  |  |  |  |  |
| 27 | Routine verifications and tests |  |  |  |  |  |  |
| 28 | Manufacturer's responsibility |  |  |  | Manufacturer's responsibility (based on applying markings) to perform routine verifications |  |  |
| 29 | Marking |  |  |  |  |  |  |
| 30 | Instructions |  |  |  |  |  |  |

| Reference No: |  |  |  |  |  |  |  |  |  | IEC 60079-7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  | Results |  |  |  |  |  |  |  |  |
|  |  | Pass | Fail | N/A |  |  | Comments |  | ExCB Review |  |
| 1 | Scope |  |  |  |  |  |  |  |  |  |
| 2 See also DS 2010/006A | Normative references |  |  |  |  |  |  |  |  |  |
| 3 | Definitions |  |  |  |  |  |  |  |  |  |
| 4 | Constructional requirements for all electrical apparatus |  |  |  |  |  |  |  |  |  |
| 4.1 | General |  |  |  |  |  |  |  |  |  |
| 4.2 | Terminals for external connections |  |  |  |  |  |  |  |  |  |
| 4.3 | Internal connections |  |  |  |  |  |  |  |  |  |
| 4.4 See also DS 2010/001 | Clearances |  |  |  |  |  |  |  |  |  |
| 4.5 See also DS 2010/001 | Creepage distances |  |  |  |  |  |  |  |  |  |
| 4.5.1 | Required values of creepage distance |  |  |  |  |  |  |  |  |  |
| 4.5.2 | Creepage distances between bare conductive parts at different potentials |  |  |  |  |  |  |  |  |  |
| 4.5.3 | Effects of ribs and grooves |  |  |  |  |  |  |  |  |  |
| 4.6 | Solid electrical insulating materials |  |  |  |  |  |  |  |  |  |
| 4.6.1 | Mechanical characteristics |  |  |  |  |  |  |  |  |  |
| 4.6.2 | Insulating parts made of plastics or laminates |  |  |  |  |  |  |  |  |  |
| 6 | Type verifications and type tests Application of tests from Clause 23 of IEC 60079-0 |  |  |  |  |  |  |  |  |  |
| 6.1 | Dielectric strength |  |  |  |  |  |  |  |  |  |
| 6.9 | Terminal insulating material tests |  |  |  |  |  |  |  |  |  |

| Reference No: |  |  |  |  |  |  |  | IEC 60079-7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Clause |  | Results |  |  |  |  |  |  |
|  |  | Pass | Fail | N/A | Comments |  | ExCB Review |  |
| 7 | Routine verification and routine tests |  |  |  |  |  |  |  |
| 7.1 | Supplementary requirements to clause 24 of IEC 60079-0 which are applicable also to type of protection increased safety "e" |  |  |  |  |  |  |  |
| 7.2 | Dielectric strength test |  |  |  |  |  |  |  |
| 8 | Marking and instructions |  |  |  |  |  |  |  |
| 8.1 | General Marking |  |  |  |  |  |  |  |
| 8.2 | Instructions for use |  |  |  |  |  |  |  |
| 8.3 | Installation instructions |  |  |  |  |  |  |  |

| Section 2 EVALUATION RECORD To be completed by ExTL IEC 60079-0 2004 and IEC 60079-7 2001 Electrical apparatus for explosive gas atmospheres - Part 0: General requirements and Part 7: Increased safety "e" |  |
| --- | --- |
| Report |  |
| Reference No. |  |
| Tested by ( + signature) Approved by (+ signature) Date of issue Content |  |
| Client |  |
| Name Address |  |
| Evaluation Record Form No. Dedicated ExTR for Exe terminal blocks Copyright reserved to the bodies participating in the IECEx Scheme |  |
| Testing laboratory |  |
| Name Address Testing location |  |
| Test item Standard IEC 60079-0 and IEC 60079-7 Description. Trademark. Model and/or type reference Manufacturer |  |
| Test Item (continued) Rating(s) Type of transformers (where relevant) Application Protection index Other characteristics Rated ambient temperature Ta (OC) | see Annex 2 n/a |
| Other information |  |
| General remarks This evaluation record is not valid unless appended to an IECEx Test Report issued by an ACB, in accordance with IECEx 02. This evaluation record shall not be reproduced except in full without the written approval of the testing laboratory. The test results presented in this evaluation record relate only to the item tested. |  |
| Copy of marking |  |

| MANUFACTURER'S DOCUMENTS (Clause 25) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Title (may be abbreviated) | Drawing No | Rev. | Issue Date |

Evaluation Record

| IEC 60079-0 |  |  |  |  |  |  |  | # COVERAGE 1/5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CLAUSE |  |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  | ### Scope |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  | ### Normative references |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  | Terms and definitions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  | Apparatus grouping and temperature classification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### Apparatus: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### Scope Electrical apparatus complying with this standard is intended for use in hazardous areas, in which explosive gas atmospheres caused by mixtures of air and gases, vapours or mists exist under normal or standard atmospheric conditions. The application of electrical apparatus outside this range may require special consideration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Operating Temperature Range of |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Standard Atmospheric Pressure Range of 80 kPa (0,8 bar) to 110 kPa (1,1 bar) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Normal Oxygen Content of 21% v/v |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Specific types of protection: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Applied to Item (apparatus or part of apparatus): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Increased safety "e" (IEC 60079-7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### Normative references (2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The electrical apparatus has been assessed to the most recent editions of the applicable standards listed in Clause 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | YES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Apparatus grouping and temperature classification (4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Group I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Groups I/II |  |  |  |  |  |  |  |  |  | Groups I/IIC |  |  |  |  |  |  |  | Groups I/IIB |  |  |  |  |  | Groups I/IIA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Group II |  |  |  |  |  |  |  |  |  | Group IIC |  |  |  |  |  |  |  | Group IIB |  |  |  |  |  | Group IIA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Specific gas/vapour (Groups I/II or Group II): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Apparatus (Group II) is marked as a function of maximum surface temperature. (Ref 5.3.2.2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## IEC 60079-0 |  |  |  |  |  |  | COVERAGE 3/5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |  |  |  |  |
| ##### CLAUSE |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1.1 |  |  |  |  |  |  | Ambient temperature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1.2 |  |  |  |  |  |  | External source of heating or cooling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2 |  |  |  |  |  |  | Service temperature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.3 |  |  |  |  |  |  | Maximum surface temperature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n/a |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  | Requirements for all apparatus |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ambient temperature (5.1.1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 20 ºC to + 40 ºC (normal range) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | special range ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ), denoted by symbol Ta ,Tamb or X (see Marking) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: Special range of ambient temperatures as stated by the manufacturer and specified in the certificate. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Service Temperature (5.2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Location |  |  |  |  |  |  |  |  |  |  |  |  |  | Minimum temperature |  |  |  |  |  |  |  |  |  |  | Maximum temperature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Component / Surface / Part) |  |  |  |  |  |  |  |  |  |  |  |  |  | Method of measurement |  |  |  |  |  |  |  |  | (ºC) |  | Method of measurement |  |  |  |  |  |  |  |  | (ºC) |  |  |  |  |  |  |  |  |
| Maximum surface temperature (5.3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Group I, maximum surface temperature (ref. manufacturer's document, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ): |  |  |  |  |
|  |  |  |  150 ºC (coal dust can form a layer) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  450 ºC (coal dust is not expected to form a layer due to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ), in which case: |  |  |  |  |  |  |  |  |  |
|  |  |  | actual maximum surface temperature ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ºC) is marked, or |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | marking includes symbol X re safe use |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirements for all apparatus (6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | The applicable requirements in the forms which follow are supplemented or modified per accompanying forms for specific types of protection or specific apparatus as noted in Clause 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### IEC 60079-0 |  |  |  |  |  |  |  |  |  | TYPE VERIFICATIONS AND TESTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |  |  |
| ###### CLAUSE |  |  |  |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  |  | ### Documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |  |  |  | ### Compliance of prototype or sample with documents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26.1 |  |  |  |  |  |  |  |  |  | ### Type tests, General |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Documentation (24) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | The manufacturer's documents that were considered in the evaluation are listed in this report |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | The manufacturer's documents specifying the design and safety aspects of the electrical apparatus address the requirements of IEC 60079-0 and those parts of IEC Publication 60079 specific to the type of protection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Compliance of prototype or sample with documents (25) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| List the prototype/sample(s) of electrical apparatus used to verify compliance with relevant requirements of this standard: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| List the prototype/sample(s) of electrical apparatus used to verify compliance with relevant requirements of standard(s) for Specific type(s) of protection: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | as above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Or |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | The prototype(s) / sample(s) of the electrical apparatus was found to comply with the listed mfr’s documents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type tests, General (26.1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Record of tests judged to be unnecessary and justification for tests omitted: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Note: Excluding Ex components which have already undergone the relevant tests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-0 |  |  |  |  |  |  |  |  |  |  | TYPE VERIFICATIONS AND TESTS 8/17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26.5.1 |  |  |  |  |  |  |  |  |  |  | ### Temperature measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Temperature measurement (26.5.1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applied to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Refer to form(s) for specific type(s) of protection ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |
|  |  | Conditions of operation: [ ] General - terminal blocks without bridges: - The test is made simultaneously on five adjacent terminal blocks connected in series by PVC-insulated conductors of the rated cross-section, as shown in Figure 2 of EN 60947-7-1. The conductors are secured according to the torque values of Table IV of EN 60947-1 or to a higher value specified by the manufacturer. The length of each of the six conductors is 1 meter for rated cross-sections up to and including 10 mm2 (AWG 8), and 2 meters for larger rated cross-sections. - The test circuit is located horizontally on a wooden surface as shown in Figure 2, the terminal blocks being securely fixed to this surface, and the conductors lying freely on it. - If the rated cross-section is below 10 mm2 (AWG 8), the conductors are rigid; for rated cross-section equal to or higher than 10 mm2 (AWG 8) the conductors shall be rigid-stranded. - The test is made with a.c. single-phase current according to the rated cross-section and is continued until steady temperature is reached. A variation of less than 2 K/hour is considered as steady temperature. - The temperature-rise of any part of the centrally located terminal block does not exceed 40 K and is measured with 3 thermocouples (terminal 1, center of current bar, terminal 2) - The test current is never higher than the current used for the normal temperature-rise test as laid down in the IEC 60947-1 Test Report Form (IECEE) of the manufacturer. [ ] General - terminal blocks with bridges: - The test is made simultaneously on six adjacent terminal blocks connected in series by PVC-insulated conductors of the rated cross-section, similar as shown in Figure 2 of EN 60947-7-1. The bridge is to be mounted between the two central terminal blocks. - The temperature-rise of the thinnest section of the bridge shall not exceed 40 K and is measured with a thermocouple. - The test procedure is identical to the test procedure for terminal blocks without bridges. See Annex 1 for the test results |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### IEC 60079-0 |  |  |  |  |  |  |  |  |  |  | TYPE VERIFICATIONS AND TESTS 12/17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |
| ###### CLAUSE |  |  |  |  |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26.8 |  |  |  |  |  |  |  |  |  |  | ### Thermal endurance to heat |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26.9 |  |  |  |  |  |  |  |  |  |  | Thermal endurance to cold |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Thermal endurance to heat (26.8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applied to |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum service temperature  75 C: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | continuous storage for |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | weeks at |  |  |  |  |  |  |  | C and |  |  | % relative humidity |  |  |  |  |  |  |  |  |
| Required: two weeks at (95  2) C and (90  5)% relative humidity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| and |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | weeks at |  |  |  |  |  |  |  | C |  |  |  |  |  |  |  |  |  |  |  |
| Required: two weeks at (20  2) K above maximum service temperature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### Thermal endurance to cold (26.9) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | continuous storage for |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | h at |  |  |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Required: 24 h at minimum service temperature, reduced by 5K to 10 K |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### IEC 60079-0 |  |  |  |  |  |  |  |  |  | TYPE VERIFICATIONS AND TESTS 17/17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |
| ###### CLAUSE |  |  |  |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |  |  | ### Routine verifications and tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Routine verifications and tests (27) (as necessary to ensure that the electrical apparatus produced complies with the documentation; also routine verifications and tests required per forms for the specific type(s) of protection) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applied to: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | List applicable specific protection type(s): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| List routine test(s): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | List applicable manufacturer’s procedure(s): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IEC 60079-0 |  |  |  |  |  |  |  |  | MARKING 1 / 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL |  |  |  |  |
| CLAUSE |  |  |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |  |  | Marking |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  | Terminal Block |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Marking Note: This sheet covers major marking content. See ExTR’s for specific types of protection for other required markings. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | listed manufacturer's documents illustrate applied markings which include the details indicated below |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Manufacturer's identification: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | *manufacturer’s name (or |  |  |  |  |  |  |  |  |  |  |  |  |  | *registered trademark) |  |  |  |  |  |  |  |  |  |  | type identification |  |  |  |  |  |  |  |  |  |  |
|  |  | serial number (except for very small electrical apparatus on which there is very limited space or connection accessories) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | batch number (alternative to serial number) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Designations: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Symbol Ex |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Symbol of type of protection “e” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Group symbol |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Operating temperature range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Maximum working voltage. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Rated conductor cross-section |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: 2. Group II electrical apparatus marked for use in a particular gas need not have a temperature reference |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Markings denoting that a certificate has been issued: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | *name/mark of certificate issuer ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) and |  |  |  |
|  |  | *certificate reference ( |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ) |  |  |  |  |  |  |  |
| Required: Last two figures of year of certification followed by serial number of certificate in that year |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ex components: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | the foregoing markings (except serial number, temperature) are included and , in addition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | *symbol U |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reduction of marking content where there is limited space: (small apparatus, extremely small apparatus, and Ex components) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Refer to foregoing markings denoted by asterisk (*) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Method of marking: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| IEC 60079-0 |  | MARKING 3 / 3 |  |  | ENCL. |
| --- | --- | --- | --- | --- | --- |
| CLAUSE |  | REQUIREMENTS |  |  |  |
| 30 |  | Instructions |  |  |  |
| Instructions (30) |  |  |  |  |  |
|  | Recapitulation of marking |  |  |  |  |
|  | Instructions for safe putting into service |  |  |  |  |
|  | Instructions for safe use |  |  |  |  |
|  | Instructions for safe assembling and dismantling |  |  |  |  |
|  | Instructions for safe maintenance |  |  |  |  |
|  | Instructions for safe installation |  |  |  |  |
|  | Instructions for safe adjustment |  |  |  |  |
|  | Training instructions as necessary |  |  |  |  |
|  | Details on anticipated operating conditions |  |  |  |  |
|  | Electrical and pressure parameters |  |  |  |  |
|  | Special conditions of use as necessary |  |  |  |  |
|  | Details on special tools as necessary |  |  |  |  |
|  | List of standards with which the apparatus complies ( |  |  | included on Certificate) |  |
|  | Instructions for replacement of cells and batteries as necessary |  |  |  |  |

| ### IEC 60079-0 |  | COMMENTS |  |
| --- | --- | --- | --- |
| #### Clause |  |  | Page |

| ## IEC 60079-7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CLAUSE |  |  |  |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  | ## Scope |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  | ##### Normative references |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  | ##### Definitions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### Scope |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The electrical apparatus defined below (see overview) and in 2/2 is feasible for coverage in accordance with the specific requirements addressed by this part of IEC 60079: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | YES (rated voltage; |  |  |  |  |  |  |  |  |  |  |  |  11 kV r.m.s. a.c. or d.c.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Notes: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: The type of protection covered by this part of IEC 60079 is dependent on the specific requirements addressed in the forms which follow. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### Normative references |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The electrical apparatus has been assessed to the most recent editions of the standards which are referenced and posted as provisions of this part of IEC 60079: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | YES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Notes: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Definitions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | All definitions covered in Clause 3 apply, as noted for the specific requirements herein |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### IEC 60079-7 |  |  |  |  |  |  |  | ##### CONSTRUCTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |  |
| ###### CLAUSE |  |  |  |  |  |  |  | ##### REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 |  |  |  |  |  |  |  | ##### General |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. 2 |  |  |  |  |  |  |  | ##### Terminals for external connections |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Terminals for external connections |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | The connection means defined below apply to internal connections |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Circuit/identification (if applicable, component mfr., part no., certification) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Apparatus rated current (A): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | See Annex 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Conductor cross-section (mm2) (ref. IEC 60364-5-523) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Number/sizes of conductors that can be safely connected; refer to document: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | aSelf-loosening from mounting(s) prevented by: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Already checked by electrical safety |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | aConductor slippage from intended location prevented by: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Already checked by electrical safety |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | a, bMeans ensuring proper contact without damage to conductors: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Already checked by electrical safety |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Means preventing turning, twisting or permanent deformation during normal tightening (tightening torque as defined by manufacturer): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Already checked by electrical safety IEC 60947-7-1,clause 8.2.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Details of resilient intermediate part included with terminals for clamping stranded conductors: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Already checked by electrical safety |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | cEffective connection for smaller conductor sizes: (mm2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Already checked by electrical safety |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Precautions against vibration and shock (i.e. where necessary): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Already checked by electrical safety according DIN EN 50155 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Precautions against electrolytic corrosion: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Already checked by electrical safety according DIN 50018 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Precautions against ferrous metal corrosion: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | a"Crimped" cable terminations are allowed if requirement is met bIncluding stranded conductors in terminals for direct conductor clamping cTerminals for connecting conductors of cross-sections  4 mm2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Terminals do not have sharp edges which would damage conductors |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Contact assured by terminals is not appreciably impaired by temperature changes in normal service |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | see forms for IEC 60079-0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | other reference: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Contact pressure is not transmitted through insulating material. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Terminals are not constructed of aluminium. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ###### IEC 60079-7 |  |  |  |  |  |  |  | CONSTRUCTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |  |  |  |  |  |  |
| ###### CLAUSE |  |  |  |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. 3 |  |  |  |  |  |  |  | ## Internal connections |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. 4 |  |  |  |  |  |  |  | ## Clearances |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. 5 |  |  |  |  |  |  |  | ## Creepage distances |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. 6 |  |  |  |  |  |  |  |  | ## Solid electrical insulating materials |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Internal connections |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Circuit/identification (if applicable, component mfr., part no., certification) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Screwed fasteners |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (locking means) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Precautions against electrolytic corrosion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clearances (4.4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The worst-case clearance comply with Table 1, based on working voltage of ............Volts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| See Annex 3 for measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Creepage distances (4.5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | material group of electrical insulating material , CTI, based on datasheet: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Group I, 600 ≤ CTI |  |  |  |  |  |  |  |  |  |  | Group II, 400 ≤ CTI < 600 |  |  |  |  |  | Group IIIa, 175 ≤ CTI < 400 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The worst-case creepage distance comply with Table 1, based on working voltage of ............Volts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| See Annex 3 for measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Solid electrical insulating materials |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Rating of material, (R) Ti based on datasheet: Material: ...................minimum ...................... ºC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Maximum operating temperature ....... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ###### IEC 60079-7 |  |  |  |  | TYPE VERIFICATIONS AND TYPE TESTS |  |  |  | ENCL. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ###### CLAUSE |  |  |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |
| 6.1 |  |  |  |  | ## Dielectric strength |  |  |  |  |  |  |  |  |  |
| 6.9 |  |  |  |  | Terminal insulating material tests |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  | ## Routine verifications and routine tests |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: The applicable test requirements of IEC 60079-0 are supplemented by tests specific to type of protection 'e' in the forms which follow. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Electric strength |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| See Annex 1 for all test results |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Terminal insulating material tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Routine verifications and routine tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Test to specific standard: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Component |  |  |  |  | Specific standard and clause | Test voltage r.m.s. a.c. or d.c. (V) | Applied between: |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | period (min) |  |  |  |  |
|  | terminal block |  |  |  |  |  | 1,2 x test voltage |  |  | 100ms |  |  |  |  |

| IEC 60079-7 |  |  | MARKING AND INSTRUCTIONS 1/3 |  |  |  |  |  |  |  |  |  |  |  |  | ENCL. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CLAUSE |  |  | REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  | Marking and instructions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 |  |  | General marking |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.3 |  |  | ## Installation instructions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: For major marking content refer to forms for IEC 60079-0, Marking. These sheets cover specific marking for the type of protection. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Marking and instructions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ref. Manufacturer's Documents list |  |  |  |  |  |  |  | ref. |  |  |  |  |  |  |  |  |  |  |
| General marking |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | rated voltage |  |  |  | rated current |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Terminals: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | conductor range |  |  |  | rated current |  |  |  |  |  |  | rated voltage |  |  |  |  |  |  |  |
| Installation instructions Required minimum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| assigned tightening torque value(s) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| indication of rearrangement or adjustment to adapt to various sizes of conductors (i.e. not otherwise obvious); |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | as marking |  |  |  |  |  |  | as instructional material |  |  |  |  |  |
| instructions for proper conductor installation (i.e. terminal construction where wiring method is not obvious) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| conductor insulation stripping requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| IEC 60079-7 |  | COMMENTS |  |  |
| --- | --- | --- | --- | --- |
| #### Clause |  |  | Page |  |

ANNEX 1

Temperature measurement:

The temperature measurement is performed at 110% of the rated current of the terminal block with rated 
cross-section and maximum cross-section.

Test conditions:

Terminal block type:				  with / without bridge type:

Rated cross-section:			 mm2 , 		 AWG

Torque value:				 Nm

Conductor length:	1 meter / 2 meter

Type of conductor:	rigid / rigid-stranded

Ambient temperature:			 °C

Test results:

Test current / temperature rise:

[ ] the test current resulting in a max. temperature rise of max. 40 K: 		 A, or

[ ] the max. temperature rise at the test current laid down in the TRF: 		 K @ 		 A