- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Title:  ExTAG/562/CD - Draft ExTAG Decision Sheet – Applicability of electrostatic 	charge requirements to headlamp straps

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document, ExTAG/562/CD Draft ExTAG Decision Sheet - Applicability of electrostatic charge requirements to headlamp straps has been prepared by UL LLC, US, and is issued for consideration by ExTAG.

In accordance with OD 035 this document is issued for a six week comment period.

Please submit comments on this new Draft DS using the comments table, a separate document, by –

2019 08 09

to

Christine Kane

Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

COLLECTION OF IECEx / ExTAG DECISION

| Standard: IEC 60079-0:2017 (Edition 7.0) IEC 60079-0:2011 (Edition 6.0) | Clause: 7.4 7.4 | Draft Decision Sheet: ExTAG/562/CD |
| --- | --- | --- |
| Subject: Applicability of electrostatic charge requirements to headlamp straps Status of document: Draft | Key words: - Electrostatic charge | - Date: 2019 06 28 Originator of proposal: UL LLC TC/SC involved: IEC/TC 31 WG 22 |
| Background It is common for devices worn on the body, such as intrinsically safe headlamps, to be sold with elastic bands to mount them on a person’s head. Question: Does a band intended to be in intimate contact with the person, and that is provided with a device which is worn on the body, need to comply with one of the options in IEC 60079-0 (such as size limitation, conductivity, or transferred charge testing) for addressing electrostatic discharge? Answer: Minimal risk of electrostatic charge buildup exists for a band which is intended to be in direct contact with the person, such as a headlamp headband. Since the headlamp is intended to be strapped to someone's head, the only charging mechanism would be through the person. Therefore clause 7.4 is not applicable to the bands, but remains applicable to the lamp enclosure. |  |  |