### INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

# Title:  ExTAG/674/CD – Draft ExTAG Decision Sheet – The number of impact test conducted on the equipment

# Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document, ExTAG/674/CD, Draft ExTAG Decision Sheet – The number of impact test conducted on the equipment has been prepared by CQM and is issued for consideration by ExTAG.

In accordance with OD 035 this document is issued for a six week comment period.

Please submit comments on this new Draft DS using the comments table, a separate document, by –

2022 05 06

to

Christine Kane

ExTAG Secretariat

| Address: IECEx Secretariat Level 33, Australia Square 264 George Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 4628 4690 Fax: +61 2 4627 5285 e-mail: info@iecex.com http://www.iecex.com |
| --- | --- |

### COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: IEC 60079-0:2017 | Clauses: 26.4.2 | Draft Decision Sheet: ExTAG/674/CD |
| --- | --- | --- |
| Subject: The number of impact conducted on one sample Status of document: Draft | Key words: - Glass Window - Fan hood - Cable Gland | # Date: March 2022 Originator of proposal: CQM TC/SC involved: IEC/SC 31M WG 1 |

BACKGROUND:

The requirements of impact test have been given in  IEC 60079-0:2017 clause 26.4.2

“The test shall be made on at least two samples of the equipment, see 26.4.1. The test shall

be made at two separate places on each sample. For equipment with light-transmitting parts

made of glass, only one of those two tests shall be on the glass.”

IEC 60079-0:2017 clause 17.2.3.2 describes additional requirement of impact on the fans, fan hoods and ventilation screen.

“The fans, fan hoods and ventilation screens shall be constructed to meet the requirements of the resistance to impact test according to 26.4.2 and the acceptance criteria given in 26.4.4.”

IEC 60079-0:2017 clause A.3.3 describes impact on the cable gland.

“ For the tests of 26.4.2, each type and size of cable gland shall be tested with the smallest mandrel or smallest cable size fitted.”

QUESTION:

For an equipment, incorporating glass window, fan hoods and integrated cable gland at the meantime, how many impact is required to comply with clause 26.4.2 of IEC 60079-0:2017?

ANSWER:

Four impacts are required. Impact should be performed on the housing itself, the glass window, fan hoods and the cable gland