INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

# TITLE: Compilation of comments and Observations on - ExTAG/679/CD – Draft ExTAG Decision Sheet – Disapplication of clause 7

# Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document contains the compilation of comments, as well as observations, from the originator, SGS Baseefa, GB, on ExTAG/679/CD, Draft ExTAG Decision Sheet – Disapplication of clause 7.

A revised document ExTAG/679A/CD – Draft ExTAG Decision Sheet – Disapplication of clause 7–has been prepared for consideration during the 2022 ExTAG Remote Meeting.

# Please inform the Secretariat immediately of any omissions or errors at-

# Christine Kane

For

ExTAG Chair Dr Lienesch

Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com

| ExCB/ ExTL | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation (to be completed by the originator) |
| --- | --- | --- | --- | --- | --- | --- |
| CMD In consultation with Intertek India, Karandikar Laboratories Pvt. Ltd., and KL Certification Services |  |  |  | After a consultation with members of the National forum and ExCBs/ExTLs from India participating in ExTAG, it is hereby stated that we have 'no comments' on draft ExTAG/679/CD. |  | Thank you |
| DEKRA / BVS DE |  |  | General | We support the proposed answer from SGS Baseefa. Additional reasons: The ignition protection type does depend on an external fan, as the fan is necessary to maintain the temperature class. The same applies analogously to fans made of light metal, for which the corresponding requirements according to cl.. 8 are also required. |  | Thank you Additional wording added |
| DNV NO |  |  | Gen | The requirements for external fan and fan housing do not lapse due to the motor being Ex d. The requirements in 60079-0 for external fan and fan housing for motors are the same if the motor is Ex d, Ex p, Ex e. |  | Thank you |
| FIDITAS HR | - | - | T | Fiditas support DS but we propose to extend the DS and clarify application of clause 7.2.1 of IEC 60079-0. We think that no common approach among ExTLs/ExCBs regarding application of thermal endurance test on external fan impellors, fan hoods, and ventilation screens manufactured from non-metallic materials | Add question 2 regarding application of thermal endurance test on external fan impellors, fan hoods, and ventilation screens manufactured from non-metallic materials | Thank you Although it was intended to be included by the current wording, specific reference to the thermal endurance testing has been included |
| FME GB |  |  | ge | We support the DS as drafted | None | Thank you |
| FMG US |  |  | ge | Support DS as drafted | None | Thank you |
| FTZU CZ |  |  | G | We agree with this draft ExTAG Decision Sheet. |  | Thank you |
| LCIE FR |  |  | General | We agree with this Draft ExTAG Decision Sheet. This DS should be published to clarify the requirements for current editions of IEC 60079-0. The subject matter should be passed to the MT60079-0 to see if a clarify would deserve to be done in the draft of IEC 60079-0 Ed. 8.0. |  | Thank you TC31 WG22 are asked to ensure clarity for the next edition of IEC 60079-0 |
| LCIE FR |  | Background | Editorial | In the first sentence, “that” is repeated twice. |  | Thank you “that that” is correct English syntax in this context (the identical words having slightly different meanings), but has been changed to “that this” to assist translation. |
| NANIO CCVE (RU) |  |  | General | NANIO CCVE supports this DS without comments. |  | Thank you |
| NCC BR | 7 17.2.2 |  |  | We agree |  | Thank you |
| NEPSI CN |  |  | G | We support the draft decision sheet ExTAG/679/CD. |  | Thank you |
| PTB DE |  |  | General | This topic opens a huge number of new questions: Clause 7 requires, for instance, also a thermal endurance test on the material. This test will only make sense if a follow up test is required, but there is no further test defined in the standard(s). | Withdraw the draft, revise to WG 22 for further discussions. Really interesting and important topic, but an IECEx DS is not an appropriate process. | TC31 WG22 have supported the draft DS TC31 WG22 are asked to ensure clarity for the next edition of IEC 60079-0 It is presumed that the impact test on the fan hood and screen would be performed after the thermal endurance testing. TC31 WG22 are asked to consider the comment in the context of the fan impellor. |
| RISE DK |  |  |  | We agree with the draft DS. If needed, we suggest this also to be further clarified in the standard IEC 60079-0 by WG 22. To not address all ignition sources and applicable requirements for a certified equipment, would seriously undermine the confidence for certified equipment (and certification services), which responsible stakeholders should be aware of. |  | Thank you Comment referred to TC31 WG22 |
| QPS CA |  |  |  | QPS supports the Decision sheet as is and has no comments |  | Thank you |
| Simtars AU |  |  |  | Simtars has no comments |  | Thank you |
| TC31/ WG22 |  |  | ge | Support DS as drafted | None | Thank you |
| TestSafe AU | Clause 7 Clause 17.2.2 of IEC 60079-0 |  | Technical | TestSafe support proposal. |  | Thank you |
| TIIS JP |  |  | Te | We support the draft DS in principle. In our understanding, external fan impellors, fan hoods and ventilation screens are not a part of enclosure, but the requirements of Clause 7 should be applied to those parts as described in 17.2.2. We do not think one is precedence and another is not. The 2nd paragraph of Answer does not seem to relate to Answer while we can understand what it says. How about moving the paragraph to Background? | Replace the last sentence of the 1st paragraph of Answer by the following sentences. The requirements of 17.2.2 are for external fans and do not relate to the Type of Protection applied to the motor. If a motor has external fans regardless of Type of Protection, 17.2.2 with the called clause should be applied. Move the 2nd paragraph of Answer to Background. | Thank you Minor adjustment made to text to reflect comment |
| ULBR BR |  |  | General | We support this proposal. |  | Thank you |
| ULD DK |  |  | General | We support this proposal. |  | Thank you |
| UL LLC US |  |  |  | UL- USA supports this decision sheet as it is written. |  | Thank you |